|  | DRAFT :1.00 |
| --- | --- |
| South Dublin County Council |  |
| Procedures for the Execution of Winter Maintenance Operations in the Roads Department of South Dublin County Council |  |
| Winter Maintenance System WMS |  |

# 1. Table of Contents

1. Table of Contents	2

1.1 Document Management	3

1.2 Glossary of Terms	3

1.3 Purpose and Scope	4

1.4 Precautionary Gritting	4

1.5 Emergency Scenarios	4

1.6 Road Weather Information System Arrangements	4

2. Depots	4

3. Health & Safety	4

3.1 Lone Working	4

4. Responsible Persons	4

4.1 IceCast Duty Engineers	4

4.2 Depot Inspectors	4

5. Procedures and Rosters	4

5.1 Duties of the Ice-Cast duty Engineer	4

5.2 Monday – Friday Procedure	4

5.3 Weekend (Sat. - Sun.) and Bank Holidays Procedure	4

5.4 Winter Maintenance Duty Engineers’ Roster	4

5.5 Rates of Spread for Precautionary Salting	4

5.6 Treatment of snow already on the road after Precautionary Salting	4

5.7 Treatment of Hard-Packed Snow and Ice	4

6 Appendices	4

6.1 Appendix 1 – Routes 1 – 8	4

6.2 Appendix 2 Emergency Scenarios	4

6.3 Appendix 3 Emergency Scenarios	4

6.4 Appendix 4 Safe Working Alone System	4

# 1.1 Document Management

| Date | Version | Author |
| --- | --- | --- |
| 01/09/2010 | DRAFT | V. Dennan |

# 1.2 Glossary of Terms

| A |  |  |
| --- | --- | --- |
| B |  |  |
| N | N.P | National Primary |
|  | N.S | National Secondary |
| S | S.D.C.C | South Dublin County Council |
| W | WMS | Winter Maintenance System |

# 1.3 Purpose and Scope

South Dublin County Council is responsible for the following

- 80 km of National Primary

- 124 km of Secondary Roads

- 689 km of Regional Road.

Winter maintenance is confined to National Primary and National Secondary Routes, Regional routes and 139km of Local roads. The percentages are summarized below in table 1.

|  | Gritted km | Total km | % |
| --- | --- | --- | --- |
| NP & NS | 80 | 80 | 100% |
| Regional | 124 | 124 | 100% |
| Local | 139 | 689 | 20.2% |

Table 1

The purpose of this document is to identify the processes, procedures and control measures employed by South Dublin County Council (S.D.C.C), to ensure that Winter Maintenance works are carried out in accordance with procedural and Health and Safety requirements. All members of staff involved with Winter Maintenance shall be fully acquainted with this Winter Maintenance Strategy and will have access to copies of it.

# 1.4 Precautionary Gritting

The primary aim is to keep these roads safe and as free as possible from winter hazards.

In order to do this, S.D.C.C Roads Department spreads salt/grit on these roads before ice or snow is expected (called Precautionary Gritting), this operation of pre-salting is timed to be completed before the onset of freezing or snowfall.

S.D.C.C gritting procedures are broken up into eight routes and are individually detailed in Appendix 1.

|  | Road | Length km |
| --- | --- | --- |
| Route 1 | N81 | 46.16 |
| Route 2 | Tallaght | 46.18 |
| Route 3 | Kimmage | 34.66 |
| Route 4 | Ballyboden | 42.67 |
| Route 5 | N7 | 48.34 |
| Route 6 | Lucan | 61.16 |
| Route 7 | Clondalkin | 38.98 |
| Route 8 | Saggart | 24.84 |

Table 2

# 1.5 Emergency Scenarios

If snow or ice settles, Snowploughing, salting and gritting is put into action. South Dublin County Council is constantly striving to improve the winter maintenance service.

In these emergency scenarios, i.e. sudden snow storms or prolonged periods of freezing temperatures. S.D.C.C engages in the following scenarios based on the amount of snow and the resources available. Each drawing of the routes below is attached in Appendices 2-10.

- S.D.C.C.WMP-Heavy Snow_1.0

- S.D.C.C.WMP-3Drivers_1.0

- S.D.C.C.WMP-4Drivers_1.0

- S.D.C.C.WMP-5Drivers_1.0

- S.D.C.C.WMP-6Drivers_1.0

- S.D.C.C.WMP-7Drivers_1.0

- S.D.C.C.WMP-LowSaltScenario1_1.0

- S.D.C.C.WMP-LowSaltScenario2_1.0

- S.D.C.C.WMP-LowSaltScenario3_1.0

# 1.6 Road Weather Information System Arrangements

An Ice Prediction System is be supplied by:

Vaisala TMI Ltd

Vaisala House

349 Bristol Road

Birmingham

Tel No:	0044 (0)121 683 1200Fax No:	0044 (0)121 683 1299

The server for the network Ice Prediction System will be housed at the Vaisala office in Birmingham.

The Ice Prediction System polls the outstations on the network at maximum intervals of one hour. This may be reduced to shorter intervals depending on conditions during the winter season.

Access to the IceNet website will be available to all the responsible Duty Engineers. Access to IceNet website is available from any computer with internet access

# 2. Depots

S.D.C.C currently operates Winter Maintenance operations from Ballymount Depot.

The new Salt Barn at Fonthill, currently under construction will be operational from XXXX

# 3. Health & Safety

S.D.C.C will ensure so far, as is reasonably practicable:

- Safe and healthy working conditions,

- Safe equipment and systems of work,

- Provision of appropriate information, instruction, training and supervision,

This procedural document should be read in conjunction with

- Roads Maintenance Ballymount – Risk Assessments / Gritters

- Roads Maintenance Palmerstown – Risk Assessments / Gritters

- Roads Maintenance Rathcoole – Risk Assessments / Gritters

A copy of which is attached in Appendix X below

# 3.1 Lone Working

Where Lone working occurs, the procedures outline in Appendix X below shall be utilised.

# 4. Responsible Persons

Winter Maintenance coordinator: Caitriona Lambert

# 4.1 IceCast Duty Engineers

| Name | Position | Route | Contact Numbers |
| --- | --- | --- | --- |
|  |  |  | Mobile |
| An Other 1 |  |  |  |
| An Other 2 |  |  |  |
| An Other 3 |  |  |  |
| An Other 4 |  |  |  |
| An Other 5 |  |  |  |

# 4.2 Depot Inspectors

| Name | Position | Route | Contact Numbers |
| --- | --- | --- | --- |
|  |  |  | Mobile |
| Isp 1 |  |  |  |
| Isp 2 |  |  |  |
| Isp 3 |  |  |  |
| Isp 2 |  |  |  |

# 5. Procedures and Rosters

A roster of Duty Engineers to give 24-hour coverage will be agreed to manage information from Met. Eireann and Vaisala. The Duty Engineer will have authority to instruct treatment as required. Consultation with the Winter Maintenance Coordinator will be carried out as necessary.

The Duty Engineer will also ensure plant and personnel are mobilised. The Duty Engineer will instruct the Inspector when a decision to treat has been made. The Inspector will ensure the drivers are instructed and ready to treat the road at the time required. The Inspector will take instruction from the Duty Engineer.

# 5.1 Duties of the Ice-Cast duty Engineer

The Pre-salting Phase will be activated by the Ice-cast Duty Engineer in the event of ice or adverse weather prediction from the Ice-cast service,. This is normally issued before 2.30 in the afternoon. Notification of the activation of the Pre-salting Phase will be given to the Plan co-ordinator, the on duty Machinery Yard Foreman and the named General Services Supervisor’s on the duty Roster of this plan.

Pre-salting shall commence at the time determined by the Ice-cast duty Engineer to ensure the completion of routes before the onset of icy conditions. As the Ice-Cast forecast and data is available at 2.30 pm therefore it is expected that the callout is given as early as possible particularly on a Friday in order that arrangements can be made before personnel finish work.

If there is no callout by 5-00 pm, then it will be assumed there is no callout unless in exceptional circumstances, such as a change in weather conditions

# 5.2 Monday – Friday Procedure

- The Ice-cast Duty Engineer checks the ice-cast and forecast at 2.30pm each day.

- In the event of the forecast of ice conditions the Duty Engineer notifies the Inspector on call before 3.30pm, of routes to be pre-salted and time of commencement.

- The Inspector then notifies the drivers and the Machinery Yard Foreman, on call for that week.

- The duty drivers proceed to their parking depots at the end of their normal working day and fit the salt spreaders.

- The Inspector notifies the loader driver, proceeds to the salt depot at the end of normal working time, and prepares the depot for loading of salt.

- At the designated time, the drivers will collect their machines, load salt and pre-salt their prearranged routes, on completion the driver will return the spreader to its parking location.

- The Inspector, will be on standby in the event of breakdown.

- From the call out time the Inspector will monitor the operations and in the event of breakdown will call out a fitter, or activate the standby salt spreader which ever he feels appropriate.

# 5.3 Weekend (Sat. - Sun.) and Bank Holidays Procedure

The pre-salting duty roster is as above

- Ice-cast Duty Engineer checks the ice-cast and forecast at or before 2.30pm each day.

- In the event of the forecast of ice conditions he notifies the Inspector on call before 3.30pm, of routes to be pre-salted and time of commencement.

- The Inspector notifies the drivers and Machinery Yard Foreman, on call for that week, of the time of callout.

- The Inspector notifies the loader driver, proceeds to the salt depot at the time appointed and prepares the depot for loading of salt.

- The duty drivers proceed to the depot at the appointed time, load salt, proceeds to complete assigned routes, and on completion the driver will return the salt spreader to its parking location.

# 5.4 Winter Maintenance Duty Engineers’ Roster

The following winter Maintenance Duty Engineers Roster is to be filled out in September of every year, detailing the dates of each Duty Engineers roster.

| Duty Engineer’s Name & Initials | Office Phone No | Mobile Tel No. |
| --- | --- | --- |
| A.n Other 1 |  |  |
| A.n Other 2 |  |  |
| A.n Other 3 |  |  |
| A.n Other 4 |  |  |
| A.n Other 5 |  |  |

| ## OCTOBER 2010 | ## Initials | ## NOVEMBER 20010 |  | ## Initials | ## DECEMBER 2009 | ## Initials |
| --- | --- | --- | --- | --- | --- | --- |
| ## Week 1 |  | ## Week 1 |  |  | ## Week 1 |  |
| ## Week 2 |  | ## Week 2 |  |  | ## Week 2 |  |
| ## Week 3 |  | ## Week 3 |  |  | ## Week 3 |  |
| ## Week 4 |  | ## Week 4 |  |  | ## Week 4 |  |
| ## JANUARY 2011 |  | ## FEBRUARY 2011 |  |  | ## MARCH 2011 |  |
| ## Week 1 |  | ## Week 1 |  |  | ## Week 1 |  |
| ## Week 2 |  | ## Week 2 |  |  | ## Week 2 |  |
| ## Week 3 |  | ## Week 3 |  |  | ## Week 3 |  |
| ## Week 4 |  | ## Week 4 |  |  | ## Week 4 |  |
| ## April 2011 |  |  |  |  |  |  |
| ## Week 1 |  |  |  |  |  |  |
| ## Week 2 |  |  |  |  |  |  |
| ## Week 3 |  |  |  |  |  |  |
| ## Week 4 |  |  |  |  |  |  |

# 5.5 Rates of Spread for Precautionary Salting

It is intended that Precautionary Action forms the major part of winter operations.

For frost and road surface temperatures at or above -2ºC salt shall be spread at 10-20 grm/m2 dependant on local conditions and the immediate forecast.

If freezing conditions are expected after rain or frost and the road surface temperature is below -2ºC spread rates will be increased to 20-40g/m2 according to the amount of moisture present and the temperature expected.  Unless freezing conditions coincide with rainfall, salting shall be delayed as long as possible to reduce loss of salt by run-off.

If continuous snow is forecast, salt shall be spread at 20-40g/m2 according to the anticipated severity of the snowfall.  Every effort will be made to ensure enough salt is applied before snow starts to stick to the road to melt the initial snowfall and to provide a wet surface.

The spread rates for precautionary salt treatments are summarised below.

| Weather Conditions | Definition | Salt Spread Rate (gram/square metre) |
| --- | --- | --- |
| Light | Frost and/or light snow | 10 |
| Moderate | Freezing conditions after rain | 20 to 30 |
| Severe | Continuous snow | 30 to 40 |

# 5.6 Treatment of snow already on the road after Precautionary Salting

Snow ploughing will commence on major routes as shown in Appendix 2 as directed by the Duty Engineer. Each pass of the plough shall be supplemented by an application of salt at a rate as per the table above. Special salting may be necessary to deal with melted water from snow, which may freeze at night, and a watch will be kept for such conditions.

# 5.7 Treatment of Hard-Packed Snow and Ice

If the previous procedures are carried out successfully then the formation of hard-packed snow should be prevented.  However, in cases where hard-packed snow and ice occur and provided that the ice is no more than 20mm thick and the air temperature is below -5ºC, then removal shall be carried out by successive salt applications of 20-40g/m2.

# 6 Appendices

# 6.1 Appendix 1 – Routes 1 – 8

| Route 1 |  |  |  |
| --- | --- | --- | --- |
|  | From | Via | To |
| N81 | M50 |  | ORR |
|  | ORR |  | Brittas |
| Mount Seskin Rd | Jobstown |  | Brittas |
| Ballinascorney | Brittas |  | Old Bawn |
| Glassamucky | Ballinascorney | Allagour | Bohernabreena |
| Old Bawn Road | Firhouse Jct |  | Kiltipper Jct |
| Kiltipper Rd | Old Bawn Rd |  | N81 |

| Route 2 |  |  |  |
| --- | --- | --- | --- |
|  | From | Via | To |
| Fortunestown Lane | N82 |  | Brookfield Rd |
| Fortunestown Way | Brookfield Rd |  | ORR |
| Brookfield Rd | Fortunestown Way |  | ORR |
| ORR | Roadstone R-about |  | N81 |
| Cookstown Rd | ORR |  | Cookstown Way R-about |
| Cookstown Rd | Cookstown Way R-about |  | Belgard Rd |
| Cookstown Way | Cookstown Rd |  | N81 |
| Old Blessington Rd | N81 |  | Cookstown Way |
| Belgard Square | All Roads between Cookstown Way and Belgard Rd |  |  |
| Belgard Rd | N7 |  | Belgard North R-about |
| Belgard Rd | Belgard North R-about |  | N81 |
| Ballymount Rd/Sylvan Drive | Belgard Rd |  | Embankment Rd |
| Embankment Rd | Belgard Rd |  | M50 |
| Mayberry Rd | Belgard Rd |  | Greenhills Rd |
| Airton Rd | Belgard Rd |  | Greenhills Rd |
| Greenhills Rd | N81 |  | M50 |
| Tymon North Rd | Greenhills Rd |  | Tallaght Rd |
| Old Blessington Rd | Greenhills Rd | Glenview R-about | N81 |
| Old Blessington Rd | Belgard Rd | Tallaght Main St | N81 |
| Tallaght Main St | Greenhills Rd Jct |  | Old Blessington Rd Jct |
| Old Bawn Rd | N81 |  | Kiltipper Rd Jct |
| Avonbeg/Seskin view Rd | N81 |  | Old Bawn Rd |
| Firhouse Rd West | Old Bawn Rd | Whitestown Way R-about | N81 |
| Whitestown/Kiltipper Way | N81 |  | Kiltipper Rd |
| Kilinarden Heigths | Firhouse Rd West |  | N81 |

| Route 3 |  |  |  |
| --- | --- | --- | --- |
|  | From | Via | To |
| Turnpike Rd | N7 |  | Ballymount Rd Lower |
| Robinhood rd | Turnpike Rd |  | Long Mile Rd |
| Ballymount Rd Upper | Ballymount Rd Lower |  | Greenhills Rd |
| Calmount Rd | M50 |  | Ballymount Ave |
| Ballymount Rd Lower | Ballymount Rd Upper |  | Greenhills R-about |
| Greenhills Rd | M50 |  | Greenhills R-about |
| Limekiln Rd | Wellington Lane | Lugnaquilla Ave | Greenhills Rd |
| St. James' Rd | Limekiln Rd |  | St. Peters Rd |
| Kimmage Rd West | Greenhills R-about | Cromwellsfort Rd | Fortfield Rd |
| Greentrees Rd | Greenhills R-about | St. Peters Rd | Wellington Lane |
| Whitehall Rd | Kimmage Rd West |  | Templeville Rd |
| Whitehall Rd West | Cromwellsfort Rd |  | Whitehall Rd |
| Wellington Lane | Greentrees Rd |  | N81 |
| Templeville Rd | Wellington Lane |  | N81 |
| Cypress Grove Rd | Kimmage Rd West | Wainsfort Rd | N81 |
| N81 | M50 |  | Old Bridge Rd |
| N81 | Old Bridge Rd |  | County Boundary |
| Spawell Firhouse Link | N81 |  | Firhouse Rd |
| Butterfield Ave | Ballycullen Rd | Firhouse Rd | Grange Rd |
| Ballyroan Rd | N81 | Old Bridge Rd | Ballybofen Rd |
| Dodder Park Rd | N81 | Springfield Rd | County Boundary |
| Fairways | Springfield Rd |  | Butterfield Ave |
| Rathfarnham Rd | County Boundary |  | Grange Rd |
| Rathfarnham Main St | Rathfarnham Rd |  | Butterfield Ave |

| Route 4 |  |  |  |
| --- | --- | --- | --- |
|  | From | Via | To |
| Grange Rd | Rathfarham Rd |  | Taylors Lane |
| Ballyboden Rd | Grange Rd | Willbrook Rd | Taylors Lane |
| Whitechurch Rd | Ballyboden Rd |  | Taylors Lane |
| Taylors Lane | Grange Rd | Ballyboden Way, Scholarstown Rd | M50 |
| Knocklyon Rd | Firhouse Rd |  | Firhouse Rd |
| Link Rd | Knocklyon Rd | The Rookery | Scholarstown Rd |
| Templeroan Rd | Knocklyon Rd |  | Scholarstown Rd |
| Scholarstown Rd | Ballyboden Way |  | Stocking Lane |
| Whitechurch Rd | Taylors Lane | Tibradden Rd | Edmondstown Rd |
| Cruagh Rd | Taylors Lane | Edmondstown Rd | Military Rd |
| Military Rd | Edmondstown Rd | Stocking Lane, Kilakee Rd | County Boundary |
| Mount Venus Rd | Edmondstown Rd |  | Kilakee Rd |
| Allenton Rd | Stocking Lane | Oldcourt Rd | Killininny Rd |
| Killininny Rd | M50 | St. Colmcilles Way | Bohernabreena Rd |
| Firhouse Rd | Killininny Rd |  | Ballycullen Rd |
| Ballycullen Rd | Firhouse Rd |  | St. Colmcilles Way |
| Ballycullen Drive | Ballycullen Rd |  | Firhouse Rd |
| Ballycullen Ave | Ballycullen Drive |  | Firhouse Rd |

| Route 5 |  |  |  |
| --- | --- | --- | --- |
|  | From | Via | To |
| N7 | City Boundary |  | Kildare Boundary |
| Longmile Rd | City Boundary |  | N7 |
| N7 Monastery Rd Slips | N7 | (Slips and Bridge Deck) | N7 |
| N7 Steelstown Slips | N7 | (Slips and Bridge Deck) | N7 |
| Link Rd | Steelstown Bridge |  | Athgoe Rd |
| Tay Lane | N7 |  | Greenogue R-about |
| Newcastle Main St | N7 | College Lane | Hazelhatch Rd |
| Hazelhatch Rd | Newcastle Village |  | Kildare Boundary |
| Peamount Rd | Newcastle Village |  | Nangor Rd |
| Loughtown Rd | Hazelhatch Rd |  | Peamount Rd |
| Baldonnel Rd | N7 |  | Nangor Rd |

| Route 6 |  |  |  |
| --- | --- | --- | --- |
|  | From | Via | To |
| N4 | City Boundary |  | Kildare Boundary |
| N4 Fonthill Slips | N4 | (combined) | Fonthill |
| N4 ORR Slips | N4 | (combined) | ORR |
| N4 Newcastle Slips | N4 | (combined) | Newcastle Rd |
| N4 Old Lucan Rd Slips | N4 | (combined) | Old Lucan Rd |
| N4 Leixlip Rd Slips | N4 | (combined) | Leixlip Rd |
| Kew Pk Service Rd | N4 |  | Leixlip Rd |
| Celbridge Rd | County Boundary |  | Leixlip Rd |
| Old Lucan Rd | N4 | N4 Underpass | N4 |
| Millstream Rd | Old Lucan Rd |  | Meadowview Grove |
| Main Street Lucan | Old Lucan Rd | The Old Hill | Old Lucan Rd |
| Main Street Lucan | Main Street Lucan |  | Old Lucan Rd |
| Lock Rd | Old Lucan Rd | Newcastle Rd | Nangor Rd |
| Nangor Rd | Lock Rd |  | ORR |
| Grange Castle Ind Est | Nangor Rd | (All of Estate) | ORR |
| ORR | Nangor Rd |  | N7 |
| ORR | Nangor Rd |  | Old Lucan Rd |
| Esker Lane | Griffeen Ave | Griffeen Rd | ORR |
| Griffeen Ave | Esker Rd |  | ORR |
| Esker Rd | Esker Lane |  | Lock Rd |
| Ballyowen Rd | Fonthill | St. Lomans Rd | ORR |
| Castle Rd | Fonthill | Lucan-Newlands Rd | ORR |
| Balgaddy Rd | Castle Rd |  | ORR |
| ORR Link Rd | ORR |  | Fonthill |

| Route 7 |  |  |  |
| --- | --- | --- | --- |
|  | From | Via | To |
| Fonthill Rd | N4 |  | N7 |
| Nangor Rd | Fonthill |  | ORR |
| Nangor Rd | Fonthill |  | N7 |
| Lindisfarne Loop Rd | Nangor Rd |  | Nangor Rd |
| Old Nangor Rd | Nangor Rd |  | Fonthill Rd |
| Hazelwood Crescent | Fonthill | St. Johns (Lawn, Rd West) | Greenisle Rd |
| Greenisle Rd | Boot Rd |  | ORR |
| ORR | N7 |  | Nangor Rd |
| Boot Rd | Fonthill |  | N7 |
| Old Nangor Rd | Fonthill |  | Tower Rd |
| Ninth Lock Rd | Fonthill | Tower Rd | Nangor Rd |
| Monastery Rd | Tower Rd | Main St Clondalkin | N7 |
| N7 Service Rd | Newlands |  | Monastery Rd |
| Woodford Walk | Monastery Rd | Woodford Hill | Nangor Rd |
| Orchard Lane | Tower Rd |  | Monastery Rd |
| Watery Lane | Orchard Lane |  | Woodford Walk |
| Lucan-Newlands Rd | Nangor Rd |  | Fonthill Rd |
| Neilstown rd | Lucan-Newlands Rd |  | Coldcut Rd |
| Cloverhill Rd | Lucan-Newlands Rd |  | Coldcut Rd |
| Coldcut Rd | Fonthill |  | Kennelsfort Rd |
| Kennelsfort Rd | Coldcut Rd |  | Old Lucan Rd |
| Palmerston Village | Stewarts Hospital | Kennelsfort Lower Jct | Roundabout |
| Liffy Valley Rd | Coldcut Rd |  | Fonthill Rd |

| Route 8 |  |  |  |
| --- | --- | --- | --- |
|  | From | Via | To |
| ORR | N7 |  | Roadstone R-about |
| N7 ORR Slips | N7 | (Slips and Bridge Deck) | N7 |
| Kingswood Ave | ORR | Old Naas Rd | N82 |
| N7 N82 Slips | N7 | (Slips and Bridge Deck) | N7 |
| N82 | N7 |  | N81 |
| Bianconi Ave | N82 |  | Garter Lane |
| Slade Rd | N7 | Garter Lane | N81 |
| Fortunestown Lane | N82 |  | Garter Lane |
| Boherboy Rd | Slade Rd |  | N81 |
| Mahons Lane | Boherboy Rd |  | N81 |
| Mill Rd | Slade Rd |  | N7 |
| Saggart One Way Lane | Garter Lane |  | Mill Lane |
| N7 Avoce Slips | N7 |  | N7 |
| Rathcoole Main St | Mill Lane | Fitzmaurice Rd | N7 |
| N7 Rathcoole Slips | N7 | (Slips and Bridge Deck) | N7 |
| Kilteel Rd | Main st |  | Windmill Hill |
| Stoney Hill Lane | Main st |  | Bolgers Lane |
| Coolmine Rd | Stoney Hill Lane | Bolgers Lane | Slade Rd |
| Green Lane | Kilteel Rd |  | Stoney Hill Lane |

# 6.2 Appendix 2 Emergency Scenarios

The following drawings to be inserted

- S.D.C.C.WMP-Heavy Snow_1.0

- S.D.C.C.WMP-3Drivers_1.0

- S.D.C.C.WMP-4Drivers_1.0

- S.D.C.C.WMP-5Drivers_1.0

- S.D.C.C.WMP-6Drivers_1.0

- S.D.C.C.WMP-7Drivers_1.0

- S.D.C.C.WMP-LowSaltScenario1_1.0

- S.D.C.C.WMP-LowSaltScenario2_1.0

- S.D.C.C.WMP-LowSaltScenario3_1.0

# 6.3 Appendix 3 Emergency Scenarios

|  |  |  | # RISK ASSESSMENT – Gritting Roads |  |  |  |  |  |  |  |  |  |  | Ref: RDS/MB/RA/0218 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### Risk Rating: Medium Risk |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: 29/01/09 |  |  |
| ## Work Activity: Gritting Roads |  |  |  |  |  |  |  |  |  |  |  |  |  | Assessed by: IRS |  |  |
| ## HAZARDS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Loading of grit by JCB (external contractor) – risk of collision with pedestrians |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reversing truck |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Mounting gritter (insertion of pins) – hand injury |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Gritter dismounting from truck while in motion |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Manual handling |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ## Persons Exposed To Risk |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Public |  |  |  |  |  | Other contractors/employees |  |  |  |  |  | Visitors |  |  |  |  |
| ## Work Description (including location) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Application of grit to public roads using gritting unit attached to truck during hours of darkness |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ## Existing Controls |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Appropriate PPE must be worn |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Gritter must be strapped to truck body by nylon straps before truck moves off Drivers specially trained in the mounting and operation of gritters |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Gritter legs must be raised and pinned before truck moves off |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ## Site Specific Controls / Recommendation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | First aid kit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Drivers to be specially trained in the mounting and operation of gritters |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  |  | IRISK ASSESSMENT |  |  |  |  |  |  |  |  |  |  |  | Ref: RDS/MB/RA/0218 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ## Monitoring Arrangements |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Monitor compliance with risk assessment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Monitor wearing of PPE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ## Supervisor Checks |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Check that PPE is being worn |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Check that new starts have undergone induction and appropriate training Check that drivers have been fully trained in the mounting and operation of gritters |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Check that safe systems of work are being followed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ##### Information, Instruction and Training |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Risk Assessment and safe systems of work |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Use of PPE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Induction training for new employees Manual handling training |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ## Personal Protective Equipment (last resort) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|     |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Additional notes on PPE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ## Initial Risk Rating (without any control measures) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Severity Rating (A) |  |  |  |  |  |  |  | Exposure Rating (B) |  |  |  |  |  | Exposure Probability Rating (C) |  |  |  |  |  |  |  |  |
| Multiple fatality |  |  |  |  |  | 15 |  | 1 Person |  |  |  | 2 |  | Exposure would rarely occur |  |  | 4 |  |  |  |  |  |
| Fatality |  |  |  |  |  | 12 |  | 2 – 5 Persons |  |  |  | 4 |  | Exposure unlikely to occur |  |  | 8 |  |  |  |  |  |
| Major injury (hospitalisation) |  |  |  |  |  | 9 |  | 6 – 20 Persons |  |  |  | 6 |  | Exposure likely to occur |  |  | 12 |  |  |  |  |  |
| Reportable injury |  |  |  |  |  | 6 |  | 21 – 100 Persons |  |  |  | 8 |  | Exposure occurs regularly |  |  | 16 |  |  |  |  |  |
| Minor accident |  |  |  |  |  | 3 |  | 100 + Persons |  |  |  | 10 |  | Exposure certain to occur |  |  | 20 |  |  |  |  |  |
| Risk Rating Calculation Risk = A X (B + C) = 192 High Risk |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Risk Reduction Rating (after controls introduced) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Severity Rating (A) |  |  |  |  | 6 |  | Exposure Rating (B) |  |  |  | 4 |  | Exposure Probability Rating (C) |  |  |  |  | 8 |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Exposure Rating (B) |  |
| Risk Rating Calculation Risk = A X (B + C) = 72 Medium Risk |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Low Risk = 18 – 59 Medium Risk = 60 – 89 Substantial Risk = 90 - 129 High Risk = 130 - 450 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ## Risk Assessment Review |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| As and when process changes or yearly |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Date of Risk Assessment………29/01/09…. ………….………………… ……….…………….…………… |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# 6.4 Appendix 4 Safe Working Alone System

# Winter Salt Spreading - Safe Working Alone System

The Salt Spreading Operations are carried to prevent the occurrence of icy conditions on roads, the main period of activity is from November 1st to March 31st, but it is not exclusive to this period.

The operation when activated is scheduled each night to finish approximately two hours before the onset of icy conditions on the road.  The equipment consists of a truck and a gritter mounted on the truck.  The gritters are located in the Machinery Yard in, and the, in The Salt used is stored in

# Hazards

The Risk Assessments of the winter salting activity refers to the activity of lone working and lists the controls as follows.

- In the event that a driver operates the Salt Spreading Unit without an attendant, a Safe Working Alone System must be put in place.

- A communication procedure and an emergency procedure must form part of the system.

- An arrangement for periodic checks must be included.

Safe Working Alone System

The following is the lone working procedure to comply with these controls

- Before leaving the depot

  - Complete check of machine

  - Complete check of controls

  - Complete communications check with designated contact

- During Route

  - Check in with designated contact at approximately 30 minute intervals or agreed timing

(Specific check in locations and timings should be agreed with the supervisor and recorded to coincide with specific points on the route eg end of specific section of road, suitable stop point, etc)

  - Check in with designated contact on completion of route

Stop Procedure

In the event of stopping during the route, the following shall be the procedure

- Park in a safe location

- Communicate with the designated contact

(When leaving the cab, inform the designated contact that you are leaving the cab, for what purpose and how long you expect to be out of the cab)

- Contact your designated contact on return to the cab

- Personnel Should never mount the Bridge of the Salt Spreader unattended

Breakdown Procedure

- In the event of breakdown during the route, the following shall be the procedure

Park in a safe location

- Communicate with the designated contact

- Remain in the cab and await instructions from your supervisor