# Comhairle Chontae Atha Cliath Theas

South Dublin County Council

MEETING OF TERENURE / RATHFARNHAM AREA COMMITTEE

TUESDAY 7th FEBRUARY 2006.

REPORT ON PART 8 LOCAL GOVERNMENT (PLANNING AND DEVELOPMENT) REGULATIONS 2001

THE PROPOSED

LIMEKILN ROAD EXTENSION

CONTENTS

- Introduction

- Consultation Process

- Written Submissions

- Submissions and Observations – Details and Responses.

- Summary

1.0	Introduction

Part 8 of the Planning and Development Regulations 2001 (S.I. No. 600 of 2001) prescribes the requirements in respect of Local Authority Development for the purposes of Section 179 of the Planning and Development Act, 2000.  The Regulations apply to the proposed works involved in the Limekiln Road Extension.

The proposed works as advertised consist of the following: -

- Construction of approximately 330 metres of 9 metre wide carriageway between Greenhills Road and Limekiln Road.

- Construction of cycletracks and footpaths.

- Installation of signal control at the junction of Limekiln Road Extension and Greenhills Road.

- Provision of pedestrian lights close to the end of the existing Limekiln Roads.

- Provision of pedestrian and maintenance entrances to Tymon Park.

- Provision of drainage and associated features.

- Provision of public lighting, road markings and signage.

2.0	Consultation Process

The proposal was advertised in accordance with Part 8 of the Planning and Development Regulations 2001 and was on public display from Thursday 14th July 2005 until Thursday 11th August 2005. Any person wishing to make a submission or observation with respect to the proposed works was invited to do so in writing.  The latest date for receipt of submissions regarding the proposed works was 8th September 2005.

3.0	Written Submissions

Eighteen written submissions, which are included in full in Appendix 1, were received from the following: -

- Roads and Traffic Department, SDCC.

- Water Maintenance Section, SDCC.

- Ms. Joan Ward.  Email, fattybells@hotmail.ie.

- Ms. Geraldine O’Reilly on behalf of Donald & Kathleen O’Reilly, 38 Limekiln Road, Manor Estate, Dublin 12.

- Water Maintenance Section, SDCC.

- Planning Department. SDCC.

- Greenpark Football Club, Paul Reid (Secretary), 64a Walnut Close, Kingswood, Dublin 24.

8.	Murphy Solicitors, Fiona Lee, Mount Clarence House, 91 Upper Georges Street, Dun Laoghaire, Co. Dublin.

9.	Greenhills Residents Association, Mary Gahan, C/O Greenhills, Community Centre, Rear St Joseph’s Road, Greenhills, Dublin 12.

- Mr. Barry J. Doyle, 2 Tibradden Grove, Greenpark, Dublin 12.

- Kevin & Theresa Duffy, 12 Kippure Ave, Greenpark, Walkinstown, Dublin 12.

- Mr. Gerry McDonnell, 4 Kippure Ave, Greenpark, Dublin 12.

- Greenpark Residents Association, Christopher Merrigan & Bernadette Hynes, 13 Glendoo Close, Greenpark, Walkinstown, Dublin 12.

- Mr. Liam Brown, 2williamb@eircom.net

- Mr. Larry Morris, 9 Lugnaquilla Ave, Green Park, Dublin 12.

- Dublin Transportation Office (DTO), Floor 3, Hainault House, 69-71 St. Stephens Green, Dublin 2.

- Councillor Cait Keane on behalf of Mr. Gerry McDonnell, 7 Kippure Avenue, Green Park, Dublin 12.

- Ms. Maureen Horan, maureen.horan@shenickgroup.com

A file containing the submissions is available at the meeting.

4.0	Submissions and Observations – Details and Responses.

Comments and Observations received from the eighteen submissions and Responses to each are as follows:

1.	Roads & Traffic Department, SDCC.

This submission states that ‘cycle facilities should be designed and provided in accordance with the current Cycle Design Manual’.

The submission also states that ‘cycle tracks are marked through junctions,

Response:

Cycle facilities have been designed and provided in accordance with the current Cycle Design Manual and to South Dublin County Council’s Standards. Detailed design of the scheme will conform to the Cycle Design Standard current at that time.

All junctions have been assessed individually.  The detailed design of each junction will be discussed with, and subject to the approval of, the Traffic Section of the Roads and Traffic Department of South Dublin County Council.

2.	Water Maintenance Section, SDCC.

- This submission states that the ‘Water Maintenance Section has no objection to the scheme subject to the recommendations of the Water Area Engineer being accommodated in the detailed design’. The submission lists water infrastructure affected by the scheme and possible action required.

Response:

All utility / service providers will be contacted at detailed design stage and impact on all services (including water services) will be identified and dealt with in conjunction with, and to the requirements of, each individual utility / service provider.

- Ms. Joan Ward.  Email, fattybells@hotmail.ie.

also

- Ms. Geraldine O’Reilly on behalf of Donald & Kathleen O’Reilly, 38 Limekiln Road, Manor Estate, Dublin 12.

These submissions concern anticipated increased traffic volumes on Limekiln Road, speed levels on the road and lack of traffic calming measures.

Response:

It is anticipated that traffic levels within Green Park Estate will reduce as a result of the proposed Limekiln Road Extension. Traffic volumes may however increase on Limekiln Road.

Traffic calming measures have recently been installed on the existing Limekiln Road.  It is proposed to extend these measures to the proposed Limekiln Road Extension, thereby restricting speed and providing a safer road environment.

5.	Water Management Section, SDCC.

- This submission concerns the same issue as Submission No. 2 and describes in greater detail requirements in relation to existing and proposed water services.

Response:

All utility / service providers will be contacted at detailed design stage and impact on all services (including water services) will be identified and dealt with in conjunction with, and to the requirements of, each individual utility / service provider.

- Planning Department, SDCC.

This submission outlines a description of the proposed scheme, its context in relation to Zoning and Road Objectives vis-à-vis the County Development Plan.

The submission further states the following:

‘The proposed Limekiln Road Extension to Greenhills Road is an objective included in the Development Plan 2004-2010 as indicated in the written statement and maps.

Road designs should have regard to the sustainable place making model as outlined in Section 11.8 – Road Design Considerations – of the Development Plan 2004-2010, if applicable.

The drawings displayed do not detail items such as landscaping and boundary treatment.  This is a detail that may need addressing to ensure that open spaces are adequately dealt with to avoid security and anti-social behaviour issues’.

Response:

The proposed scheme involves the extension of Limekiln Road to Greenhills Road.  The scheme has and will have regard to the sustainable place making model as outlined in Section 11.8 – Road Design Considerations – of the Development Plan 2004-2010, where applicable.

Details such as landscaping will be to South Dublin County Council standards. New boundaries in Tymon Park will be similar to existing boundaries currently in place.

- Greenpark Football Club, Paul Reid (Secretary), 64a Walnut Close, Kingswood, Dublin 24.

This submission concerns the playing fields to the east of the proposed Limekiln Road Extension and the future of this area.

Response:

The Roads Department of South Dublin County Council proposes to construct the Limekiln Road Extension to the layout shown on the Part 8 Display Drawing.  The proposed road alignment does not impact on the existing playing pitches and therefore there will be no change to the current status of this area. The area in question is zoned Objective GB – to preserve a ‘Green Belt’ between development areas. There are no plans to change this zoning status.

8.	Murphy Solicitors, Fiona Lee, Mount Clarence House, 91 Upper Georges Street, Dun Laoghaire, Co. Dublin.

This submission has been made on behalf of William and Louise Walters of Tymon, Greenhills Road, Walkinstown, Dublin 12.

The submission states that the proposed road will be approximately 30m from the rear of the property and raises concerns in relation to:

- Noise pollution.

- Emissions from cars.

- Noise pollution during construction.

- Effect of road lighting.

- Effect on the public park, particularly the sports fields.

The submission also states that there was an illegal dump on site, approximately 30 years ago and raises concerns over the possible uncovering of the dump.

The submission also states that ‘if the roadway is constructed that it will be necessary for them to make alterations to their property to include possible double glazing, if the noise level is of an intensive nature’.

Response:

The proposed road is 32m from the rear of the property and is in a cutting, which assists in terms of noise abatement.  A low wall and railing is proposed along the boundary of the road, which will also assist in terms of noise abatement.  Given the distance between the property and the proposed road and the level of the road it is highly unlikely that noise levels at the property will exceed acceptable levels.  A study will however be undertaken to ascertain noise impact on the property.  If the noise level exceeds acceptable limits, then a low noise surface will be installed and if necessary the boundary wall will be increased to an appropriate height as a mitigation measure.

Emissions from vehicular traffic will have no impact on the property.

Normal construction activities are generally restricted to the period between 8.00am and 7.00pm, Monday to Friday and between 9.00am and 4.30pm on Saturdays. This excludes the pumping out of excavations, security and emergency works.  It will be a requirement of the Construction Contract that noise control measures be in place at all times e.g. construction vehicles and equipment to be properly maintained, equipment used intermittently to be shut down or throttled back to a minimum during periods when not in use, all vehicles and equipment to be fitted with exhaust silencers, where appropriate.  Dust control measures will also be implemented to minimise dust emanating from the construction site.

Lighting on the proposed road will be positioned so that the direction of light is away from the property.

The Roads Department of South Dublin County Council proposes to construct the Limekiln Road Extension to the layout shown on the Part 8 Display Drawing. The proposed road alignment does not impact on the existing playing pitches and therefore there will be no change to the current status of this area.

Investigations will be carried out to ascertain ground conditions along the proposed route of the road.  If illegally dumped material is encountered it will be properly dealt with by removal to a licensed landfill.

9.	Greenhills Residents Association, Mary Gahan, c/o Greenhills, Community Centre, Rear St Joseph’s Road, Greenhills, Dublin 12.

This submission raises concerns in relation to increased traffic volumes on Limekiln Avenue travelling from Greenhills Road to St. Peters Road. It states that ‘at present the residents of St. James Road and residents of the roads leading off St. James Road have serious difficulties entering the stream of traffic on St. James Road.

The submission also states that the Tymon Park is ‘an amenity for the people and not a facility for use as a solution to the traffic congestion/free flow problem of South County Dublin’.

Response:

It is anticipated that the Greenhills Ballymount Reconfiguration will have little impact on through-traffic within Green Park.  However, it is anticipated that the Limekiln Road Extension will reduce the extent of ‘rat-running’ that currently exists within Green Park.

Traffic calming measures are currently being installed on the existing Limekiln Road.  It is proposed to extend these measures to the proposed Limekiln Road Extension, thereby restricting speed and providing a safer road environment.  This will also make this route less attractive to ‘rat-running’ traffic that currently utilises this route.

The Scheme is an objective of the South Dublin County Development Plan 2004 – 2010. The various elements of the Scheme form the following Objectives of the Development Plan 2004 – 2010:

- The realigned road is a 6-year Roads Objective.

- The provision of cycle facilities is a proposed Cycle Route Network Objective.

The Scheme also forms part of the Integrated Framework Plan for Land Use and Transportation for the Tallaght Area (IFPLUT).  The Plan, finalised in 2003, was commissioned by South Dublin County Council and the Dublin Transportation Office (DTO).  Its purpose was to establish a strategy to maximise the potential benefit of sustainable development of the area and to fully utilise proposals for improving public transport and highway systems, together with cycling and walking facilities.  The Limekiln Road Extension forms part of the proposed cycle and road network improvements examined under IFPLUT.

- Mr. Barry J. Doyle, 2 Tibradden Grove, Greenpark, Dublin 12.

This submission concerns both the Greenhills Ballymount Reconfiguration Scheme and the Limekiln Road Extension Scheme.  The submission also raises issues in relation to traffic calming measures within Green Park Estate.  Issues relating to the Limekiln Road Extension are as follows:

- That there be no change to the status of the playing pitches in Tymon Park.

- That a pedestrian bridge be installed instead of the proposed pedestrian crossing.

- That traffic lights be installed at the junction of Kippure Avenue and Limekiln Road.

- That local residents and community groups be kept informed of any alterations or changes to the developments already planned.

The submission also states that ‘as a resident I am in favour of any plan to ease traffic in the estate and improve our quality of life’.

Response:

- The Roads Department of South Dublin County Council proposes to construct the Limekiln Road Extension to the layout shown on the Part 8 Display Drawing.  The proposed road alignment does not impact on the existing playing pitches and therefore there will be no change to the current status of this area.

- A pedestrian bridge is not warranted at this location from either a pedestrian safety or pedestrian numbers viewpoint. A signalised pedestrian crossing close to the end of the existing Limekiln Road is proposed as part of the scheme

- Traffic levels do not warrant traffic lights at this location.  Traffic calming measures recently installed on Limekiln Road and similar measures on the proposed Limekiln Road Extension will assist traffic at this junction.  It is however proposed to include ducting at this location in order that traffic lights can be fitted at a later date, if the need arises.

- There are no changes proposed to the scheme.

- Kevin & Theresa Duffy, 12 Kippure Ave, Greenpark, Walkinstown, Dublin 12.

This submission makes general observations in relation to traffic and traffic congestion within Green Park Estate.  The submission states that ‘having regard to the foregoing the proposed development is welcome and has the potential to ameliorate the problems highlighted.  For that reason we fully support the Council’s proposals’.

The submission also states that ‘the effectiveness of the proposed scheme in reducing traffic volumes through the estate will be significantly impaired unless the present entrance to Green Park from Greenhills Road is closed’.

Response:

The closure or non-closure of the entrance to Green Park from Greenhills Road via the ‘dip’ is associated with the Greenhills Ballymount Reconfiguration Scheme.  The majority of submissions in relation to the Greenhills Ballymount Reconfiguration Scheme requested that this link be kept open.

It is anticipated that the Limekiln Road Extension will significantly reduce the volume of traffic accessing the Greenhills Road via the ‘dip’.  The access to Green Park to/from Greenhills Road via the ‘dip’ will remain open to traffic as part of the Greenhills Ballymount Reconfiguration Scheme.

- Mr. Gerry McDonnell, 4 Kippure Ave, Greenpark, Dublin 12.

This submission concerns both the Greenhills Ballymount Reconfiguration Scheme and the Limekiln Road Extension Scheme.  Issues relating to the Limekiln Road Extension are as follows:

- Through Traffic.

The current traffic set-up into, within and exiting Green Park is discussed including the extent of ‘rat-running’ through the estate and the resultant ‘major negative impact on our quality of life in this area’.

The submission seeks clarification in how ‘the Council, in implementing the above proposed schemes, proposes to either eliminate or to substantially reduce the amount of through traffic through this estate while at the same time facilitating residents in their usage of the estate and its environs’.

2)	Public Open Spaces.

The question of the future status of the portion of Tymon Park to the east of the proposed Limekiln Road Extension is queried and it is stated that this land should remain in the full ownership of the County Council ‘as a public open space for recreational and sporting purposes’.

Response:

- It is anticipated that the Limekiln Road Extension will reduce the extent of ‘rat-running’ that currently exists within Green Park.

- The Roads Department of South Dublin County Council proposes to construct the Limekiln Road Extension to the layout shown on the Part 8 Display Drawing.  The proposed road alignment does not impact on the existing playing pitches and therefore there will be no change to the current status of this area. The area in question is zoned Objective GB – to preserve a ‘Green Belt’ between development areas. There are no plans to change this zoning status.

- Greenpark Residents Association, Christopher Merrigan & Bernadette Hynes, 13 Glendoo Close, Greenpark, Walkinstown, Dublin 12.

This submission concerns both the Greenhills Ballymount Reconfiguration Scheme and the Limekiln Road Extension Scheme. In relation to Limekiln Road Extension the Association states that it opposes all aspects of the proposed Scheme until:

      - South Dublin County Council provide detailed information as to how they expect the plan to impact the residents of Greenpark in relation to:

- Traffic volumes through Greenpark estate and the Limekiln Road.

      - South Dublin County Council  provides ‘cast iron’ guarantees that the proposed area of Tymon Park adjacent to Greenpark, to be sectioned by the proposed Limekiln Road Extension, will remain a GREEN designated area unless otherwise agreed by a majority of Greenpark Residents.

      - Concerns about volumes of traffic through Greenpark Estate from Limekiln/Templeogue direction to the proposed new supermarket are addressed.

      - Sufficient time is allocated by South Dublin County Council  to allow residents achieve consensus on outstanding issues relative to the proposed roads adjustments.

      - The submission also states that the ‘period allocated by South Dublin County Council  for consultation i.e. 6 weeks (extended to 8 weeks) has proved totally inadequate’.

Response:

            - It is anticipated that the Greenhills Ballymount Reconfiguration will result in little, if any, change to traffic volumes through Green Park Estate.  The Limekiln Road Extension is however expected to reduce through-traffic within Green Park Estate.  Traffic volumes may however increase on Limekiln Road.

            - The Roads Department of South Dublin County Council proposes to construct the Limekiln Road Extension to the layout shown on the Part 8 Display Drawing.  The proposed road alignment does not impact on the existing playing pitches and therefore there will be no change to the current status of this area. The area in question is zoned Objective GB – to preserve a ‘Green Belt’ between development areas. There are no plans to change this zoning status.

            - It is considered that the Limekiln Road Extension has little relevance to traffic associated with the new supermarket that travels through Green Park Estate.  It is considered that such traffic will be in place irrespective of whether the proposed realignment scheme is implemented or not.

4 & 5.	The requirement of the Planning and Development Regulations 2001 (S.I. No. 600 of 2001) in relation to Part 8 Planning procedures is as follows:

- Plans and particulars to be available for inspection for a period of not less than four weeks.

- Submissions or observations with respect to the proposed development may be made in writing before a specified date, which shall not be less than 2 weeks after the period for inspection of plans and particulars.

The plans and particulars were on official display from Thursday 14th July 2005 until Thursday 11th August 2005, a period of four weeks in compliance with the Regulations.

The latest date for receipt of submissions regarding the proposed works was 8th September 2005, a period of four weeks after the period for inspection of plans and particulars.

14.	Mr. Liam Brown, 2williamb@eircom.net

This submission concerns the following:

      - Increased traffic volumes to Limekiln Road

      - Present difficulties for residents exiting and entering their homes.

      - Dangers to children entering and exiting St. Peters School and in crossing Limekiln Road.

      - Traffic noise associated with the M50 will be increased.

      - The necessity in due course to construct ramps on the road.

      - Increased dangers to the public who wish to gain access to Tymon Park.

      - The present route to Greenhills Road through Green Park Estate regulates the speed at which traffic travels along the road and that the present ‘routes of access are sufficient to serve the needs of the public’.

Response:

It is anticipated that traffic levels within Green Park Estate will reduce as a result of the proposed Limekiln Road Extension. Traffic volumes may however increase on Limekiln Road.

Traffic calming measures are currently being installed on the existing Limekiln Road.  It is proposed to extend these measures to the proposed Limekiln Road Extension, thereby restricting speed and providing a safer road environment.

The existing volume of traffic that ‘rat-runs’ through Green Park Estate has been a major cause of concern to residents within the estate for many years.  The Limekiln Road Extension is a 6-years Roads Objection of the Development Plan 2004-2010 and is required to control and regulate traffic in the general vicinity of Green Park.

Noise associated with the Limekiln Road Extension will be minimal and traffic calming measures and a low noise surface will mitigate the impact.

- Mr. Larry Morris, 9 Lugnaquilla Ave, Green Park, Dublin 12.

This submission concerns the following:

- No personal contact was made with residents of Green Park.

- The proposal is for the convenience of more quality bus corridors and the movement of industrial traffic from as far as the Long Mile Road.

- That an environmental impact study should be carried out to ‘allow the general concerns of residents to be heard with regard to access, public transport, consumption of traffic, splitting of a section of public park and wildlife that reside in the park’.

- The future plans being considered for the section of park ‘sheared off from Tymon Park’.

Response:

      - The requirement of the Planning and Development Regulations 2001 (S.I. No. 600 of 2001) in relation to Part 8 Planning procedures is as follows:

- Plans and particulars to be available for inspection for a period of not less than four weeks.

- Submissions or observations with respect to the proposed development may be made in writing before a specified date, which shall not be less than 2 weeks after the period for inspection of plans and particulars.

The plans and particulars were on official display from Thursday 14th July 2005 until Thursday 11th August 2005, a period of four weeks in compliance with the Regulations.

The latest date for receipt of submissions regarding the proposed works was 8th September 2005, a period of four weeks after the period for inspection of plans and particulars.

A presentation was made to the Green Park Residents Association on Tuesday 16th August 2005.  The presentation involved a detailed description of the Limekiln Road Extension and the Greenhills Ballymount Reconfiguration Scheme followed by questions and answers.

      - There are no bus lanes on the proposed Limekiln Road Extension.  The proposed extension is independent of the bus lanes proposed on the Greenhills Ballymount Reconfiguration Scheme.

      - The scheme is not of sufficient scale or content that would require the preparation of an EIS under current legislation.

      - The Roads Department of South Dublin County Council proposes to construct the Limekiln Road Extension to the layout shown on the Part 8 Display Drawing.  The proposed road alignment does not impact on the existing playing pitches and therefore there will be no change to the current status of this area.

- Dublin Transportation Office (DTO).

The DTO submitted one single submission covering five separate Part 8 schemes, namely a) The Greenhills Road Realignment at Parkview, b) Greenhills/Ballymount Reconfiguration, c) Limekiln Road Extension, d) Robinhood/Ballymount Reconfiguration and e) Embankment Road Extension.

The submission makes a general comment as follows:

‘The DTO supports three of the above schemes in principle as they are related to projects referred to in the DTO Strategy “A Platform for Change” under the category ‘Non-national Roads Projects – Metropolitan Area. These are a) The Greenhills Road Realignment at Parkview, b) Greenhills/Ballymount Reconfiguration and c) Embankment Road Extension’.

DTO comments particular to the Limekiln Road Extension scheme are as follows:

            - An initial examination of all these schemes taken together would indicate the provision of a new and interrelated orbital and radial route in the South Dublin County Council area.  In particular, the Robinhood extension, the Limekiln extension and the reconfiguration of the Greenhills Road would appear to provide for a new orbital route between the Long Mile Road/Naas Road and Wellington Lane, which connects with the N81.

            - The DTO would recommend that an assessment be undertaken which takes all of the schemes into account (assessment elements are detailed).

            - The DTO would welcome the opportunity to discuss certain design elements with the local authority, in particular

- Road cross-section and lane width.

- Continuity of bus and cycle provision.

- Cycle provision through junctions.

- Pedestrian crossing facilities.

- The interaction between HGV’s and vulnerable road users’.

Response:

1.	The proposed roads do form a connection between the Naas Road and Wellington Road, as do existing roads.  The proposed roads will not be to the standard of an ‘orbital’ road and will therefore not have the capacity for such a use.

2.	The Scheme has been a Roads Objective in numerous County Development Plans.  It is an objective of the South Dublin County Development Plan 2004 – 2010.  The various elements of the Scheme form the following Objectives of the Development Plan 2004 – 2010:

- The Limekiln Road Extension is a 6-year Roads Objective.

- The provision of cycle facilities is a proposed Cycle Route Network Objective.

The Scheme also forms part of the Integrated Framework Plan for Land Use and Transportation for the Tallaght Area (IFPLUT).  The Plan, finalised in 2003, was commissioned by South Dublin County Council and the Dublin Transportation Office (DTO).  Its purpose was to establish a strategy to maximise the potential benefit of sustainable development of the area and to fully utilise proposals for improving public transport and highway systems, together with cycling and walking facilities.  The Limekiln Road Extension forms part of the proposed cycle and road network improvements examined under IFPLUT.

3.	The Scheme has been designed to current road standards.  South Dublin County Council will discuss appropriate design elements with the DTO.

17.	Councillor Cait Keane on behalf of Mr. Gerry McDonnell, 7 Kippure Avenue, Green Park, Dublin 12.

This submission attached correspondence received by Councillor Cait Keane. Refer to Submission No. 12 in relation to the submission and Response.

18.	Ms. Maureen Horan, maureen.horan@shenickgroup.com

This submission makes comments in relation to the Limekiln Road Extension and the Greenhills Ballymount Reconfiguration Scheme.  In relation to the Limekiln Road Extension the submission requests that traffic lights be installed at the junction of Green Park Estate and Limekiln Road.

Response:

Traffic levels do not warrant traffic lights at this location.  Traffic calming measures recently installed on Limekiln Road and similar measures on the proposed Limekiln Road Extension will assist traffic at this junction.

It is proposed to include ducting at this location in order that traffic lights can be fitted at a later date, if the need arises.

5.0	Summary

The various works proposed under the Limekiln Road Extension are in accordance with the 2004-2010 County Development Plan and with the proper Planning and Development of the area.

It is therefore proposed to proceed with the Limekiln Road Extension subject to the following:

- The inclusion of traffic calming measures on the Limekiln Road Extension, as an extension of the recently installed traffic calming measures on the existing Limekiln Road.

- The inclusion of ducting at the junction of Limekiln Road and Green Park Estate to enable traffic lights to be easily installed if they are deemed to be required at a future date.

The scheme will be funded from Development Levies.

A slide is available at the meeting.