COMHAIRLE CHONTAE ÁTHA CLIATH THEAS

SOUTH DUBLIN COUNTY COUNCIL

MEETING OF LUCAN LOCAL AREA COMMITTEE

Tuesday, 26th September, 2017

HEADED ITEM No. 11

REPORT UNDER SECTION 38 OF THE ROAD TRAFFIC ACT 1994

PHASE 2 OF THE N4 TO CITY CENTRE CYCLE AND PEDESTRIAN IMPROVEMENTS SCHEME

##### INTRODUCTION:

Section 38 of the Road Traffic Act 1994 empowers a Roads Authority to provide traffic calming measures in respect of public roads in their charge

DESCRIPTION OF SCHEME

The proposed works will be carried out from the R136 (Outer Ring Rd)/Willsbrook Rd junction to the R136 (Outer Ring Rd)/Old Lucan Rd junction.  The scheme aims to improve the public realm by providing upgraded footpaths and cycle tracks, additional landscaping, improved signage and new signalized crossing locations on the approaches to the R136 (Outer Ring Rd)/Old Lucan Rd junction. The cycle facilities will join to existing cycle facilities on Willsbrook Rd and the R136 (Outer Ring Rd) that are heavily used by school pupils attributed to several primary and secondary schools located near the proposed scheme and also to commuters using the existing N4 cycle route to the City Centre.

The proposed works will comprise the following:

- The removal of the right turn from R136 (Outer Ring Rd) (southbound) onto the N4 on-ramp (westbound), in order to provide a segregated cycle route in both directions across the bridge over the N4. This will allow for the separation of buses and cyclists, thereby increasing safety for cyclists.

- The extension of the right turning lane from R136 (Outer Ring Rd) (southbound) on to Willsbrook Rd in order to increase capacity and prevent delays to southbound traffic.

- The provision and improvement of almost 1.3km of two way off road cycle track from the R136 (Outer Ring Rd)/Willsbrook Rd junction to the R136 (Outer Ring Rd)/Old Lucan Rd junction.

- Improved crossing facilities on all arms of the R136 (Outer Ring Rd)/Old Lucan Rd junction.

- Bus stop improvements with Real Time Passenger Information (RPTI) and improved set-down facilities, including setting back the existing bus stop on the south side of Old Lucan Rd, which currently blocks traffic.

- Additional landscaping works throughout the scheme

- Provision of new public lighting, road markings and cycle route signage

CONSULTATION PROCESS:

Details of the proposed Cycling and Pedestrian Improvements Scheme was advertised in the Lucan Gazette on June 1st 2017.  The closing date for submissions was 5.00 p.m. on Thursday 1st July 2017.

SUBMISSIONS RECEIVED

One Hundred and ninety five (195) submissions were received.

A schedule of the submissions is attached to this report and a file containing the submissions is available at the meeting.

ISSUES RAISED AND ROADS DEPARTMENT REPLIES TO THE SUBMISSIONS:

|  | Submission by Resident (1), (7), (13) |  |
| --- | --- | --- |
| 1. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Removal of right turn will force traffic through areas such as Willsbrook intersection and Lucan Village - Removal of right turn will have a disastrous effect on residents of Lucan. |
| 1. | Response | - Noted - This scheme is being designed to bring some measure of choice for the residents of Lucan and to encourage parents to allow their children to cycle to school rather than being driven. The proposed cycle network produced by the NTA shows a primary cycle route along the N4 to the City Centre. There is a proposed secondary route along the ORR to join with the N4 primary route, which is part of this scheme. - This scheme will provide a safe route for school children to cycle to the large number of schools in Lucan village and commuters to the primary route to the City Centre along the N4. The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded. |
|  | Submission by Residents (2) |  |
| 2. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Removal of right turn will force traffic through areas such as Willsbrook intersection and Lucan Village. - Supports the building of new cycle lanes and bus clear lanes for the city centre - The Proposed Cycle Lane would get a minimal amount of Cyclists due to its peripheral location on the outskirts of the village. |
| 2. | Response | - Noted - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded. - Noted - This scheme is being designed to bring some measure of choice for the residents of Lucan and to encourage parents to allow their children to cycle to school rather than being driven. The proposed cycle network produced by the NTA shows a primary cycle route along the N4 to the City Centre. There is a proposed secondary route along the ORR to join with the N4 primary route, which is part of this scheme. This scheme will provide a safe route for school children to cycle to the large number of schools in Lucan village and commuters to the primary route to the City Centre along the N4. |
|  | Submission by Resident (3) |  |
| 3. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Notes the high volume of traffic travelling through Lucan Village currently - The Proposed Cycle Lane would get a minimal amount of Cyclists due to its peripheral location on the outskirts of the village. |
| 3. | Response | - Noted - Noted - This scheme is being designed to bring some measure of choice for the residents of Lucan and to encourage parents to allow their children to cycle to school rather than being driven. The proposed cycle network produced by the NTA shows a primary cycle route along the N4 to the City Centre. There is a proposed secondary route along the ORR to join with the N4 primary route, which is part of this scheme. This scheme will provide a safe route for school children to cycle to the large number of schools in Lucan village and commuters to the primary route to the City Centre along the N4. The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded. |
|  | Submission by Resident (4) |  |
| 4. | Submission | - Objects to proposed works to be carries out under “Phase 2 of N4 City Centre cycle and Pedestrian Improvements Scheme” - Removal of right turn will lead to nearby neighbourhoods being used by traffic to gain access onto the N4. - Current cycle lanes facilitates are adequate due to low numbers of cyclists in the area. |
| 4. | Response | - Noted - The change in traffic behaviour will be monitored by the Roads Dept and any undesirable behaviour will be discouraged by traffic management measures. - This scheme is being designed to bring some measure of choice for the residents of Lucan and to encourage parents to allow their children to cycle to school rather than being driven. The proposed cycle network produced by the NTA shows a primary cycle route along the N4 to the City Centre. There is a proposed secondary route along the ORR to join with the N4 primary route, which is part of this scheme. This scheme will provide a safe route for school children to cycle to the large number of schools in Lucan village and commuters to the primary route to the City Centre along the N4. Whilst cycle numbers may not be significant at present, providing high quality cycle facilities will help to increase cycle figures in the area. This has already been shown to be the case, the recent CSO figures has shown an increase in cycling of 43% between 2011 and 2016. |
|  | Submission by Resident (5), (6), (9), (10), (11), (14), (16), (17), (20), (21), (22), (24), (29), (27), (30), (35), (37), (40), (42), (47), (50), (51), (55), (69), (71), (76), (78), (79), (82), (92), (94), (95), (96), (97), (98), (100), (142), (151), (152), (184), (185) |  |
| 5. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Raises concerns regarding traffic volumes making the right turn evening peak time. - Removal of right turn will significantly increase traffic volumes driving through Lucan Village. - Removal of right turn will force traffic to join N4 at the congested Newcastle road (Junction 4) N4/R120 - Banning the right turn would cause excess traffic and lead to congestion in roads around housing estates and endanger the lives of children as it would significantly increase traffic around schools - Notes the high volume of traffic travelling through Lucan Village currently - Removal of right turn will force traffic through areas such as the already gridlocked Willsbrook Road. - The Proposed Cycle Lane would get a minimal amount of Cyclists due to its peripheral location on the outskirts of the village. |
| 5. | Response | - Noted - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded. - Please see response No.2 - Please see response No.2 - This scheme is being designed to bring some measure of choice for the residents of Lucan and to encourage parents to allow their children to cycle to school rather than being driven. The proposed cycle network produced by the NTA shows a primary cycle route along the N4 to the City Centre. There is a proposed secondary route along the ORR to join with the N4 primary route, which is part of this scheme. This scheme will provide a safe route for school children to cycle to the large number of schools in Lucan village and commuters to the primary route to the City Centre along the N4. Any detrimental change in traffic behaviour will be monitored by the Roads Dept and any undesirable behaviour will be discouraged by traffic management measures - Please see response No.5 - Please see response No.5 - Whilst cycle numbers may not be significant at present, providing high quality cycle facilities will help to increase cycle figures in the area. This has already been shown to be the case, the recent CSO figures has shown an increase in cycling of 43% between 2011 and 2016. |
|  | Submission by Resident (8) |  |
| 6. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Removal of right turn will force traffic through Ballyowen to Liffey Valley increasing traffic volumes in the area. - Removal of right turn will force traffic through areas such as the already gridlocked Willsbrook Road. - Submission outlines existing cycle lane in Lucan Area are rarely used. The majority of cyclists use the road network. - Lucan/Adamstown is a massively growing community and believes N4/R136 Junction is integral to its growth. |
| 6. | Response | - Noted - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded. - Please see response No.2 - This scheme is being designed to bring some measure of choice for the residents of Lucan and to encourage parents to allow their children to cycle to school rather than being driven. The proposed cycle network produced by the NTA shows a primary cycle route along the N4 to the City Centre. There is a proposed secondary route along the ORR to join with the N4 primary route, which is part of this scheme. This scheme will provide a safe route for school children to cycle to the large number of schools in Lucan village and commuters to the primary route to the City Centre along the N4. Whilst cycle numbers may not be significant at present, providing high quality cycle facilities will help to increase cycle figures in the area. This has already been shown to be the case, the recent CSO figures has shown an increase in cycling of 43% between 2011 and 2016. - Noted |
|  | Submission by Resident (12) |  |
| 7. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Notes the high volume of traffic travelling through Lucan Village currently - Removal of right turn will force traffic through areas such as the already gridlocked Willsbrook Road. - Removal of right turn will force traffic to join N4 at the congested Newcastle road (Junction 4) N4/R120 - Lucan area need had a better public transport system, cars are the only way to reliably get around Lucan and its environs. - Banning the right turn would cause excess traffic and lead to congestion in roads around housing estates and endanger the lives of children as it would significantly increase traffic around schools. |
| 7. | Response | - Noted - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded. - Please see response No.2 - Please see response No.2 - The provision of public transport is outside the capacity of this scheme. However this scheme is being designed to bring some measure of choice for the residents of Lucan and to encourage parents to allow their children to cycle to school rather than being driven. The proposed cycle network produced by the NTA shows a primary cycle route along the N4 to the City Centre. There is a proposed secondary route along the ORR to join with the N4 primary route, which is part of this scheme. This scheme will provide a safe route for school children to cycle to the large number of schools in Lucan village and commuters to the primary route to the City Centre along the N4. - The change in traffic behaviour will be monitored by the Roads Dept. and any undesirable behaviour will be discouraged by traffic management measures |
|  | Submission by Resident (18) |  |
| 8. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Removal of right turn will force traffic through areas such as the already gridlocked Willsbrook Road. - Cycle Lanes Should be integrated to fit in with current traffic layouts - How will traffic access N4 Westbound? - Removal of right turn will force traffic to join N4 at the congested Newcastle road (Junction 4) N4/R120. - Banning the right turn would cause excess traffic and lead to congestion in roads around housing estates and endanger the lives of children as it would significantly increase traffic around schools. |
| 8. | Response | - Noted - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded. - Please see response No.2 - The N4 Westbound traffic will either travel through Lucan Village to the Newcastle interchange or else travel through Willsbrook Rd to this interchange. - Please see response No.2 - The change in traffic behaviour will be monitored by the Roads Dept. and any undesirable behaviour will be discouraged by traffic management measures |
|  | Submission by Resident (19) |  |
| 9. | Submission | - Notes the high volume of traffic travelling through Lucan Village currently - Submission outlines existing cycle lanes in the area are rarely used, so an extension cannot be justified. - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Removal of right turn will force traffic to join N4 at the congested Newcastle road (Junction 4) N4/R120. - Lucan/Adamstown is a massively growing community and believes N4/R136 Junction is integral to its growth. - Removal of right turn will force traffic through areas such as the already gridlocked Willsbrook Road - Health and Safety concern regarding motorist may perform U-turn at the entrance to a housing estate - Lucan area need had a better public transport system. - Submission suggest South Dublin County Council should investigate more orbital road, to ease the commuter traffic between the N3/N4 and the N7. |
| 9. | Response | - Noted - This scheme is being designed to bring some measure of choice for the residents of Lucan and to encourage parents to allow their children to cycle to school rather than being driven. The proposed cycle network produced by the NTA shows a primary cycle route along the N4 to the City Centre. There is a proposed secondary route along the ORR to join with the N4 primary route, which is part of this scheme. This scheme will provide a safe route for school children to cycle to the large number of schools in Lucan village and commuters to the primary route to the City Centre along the N4. Whilst cycle numbers may not be significant at present, providing high quality cycle facilities will help to increase cycle figures in the area. This has already been shown to be the case, the recent CSO figures has shown an increase in cycling of 43% between 2011 and 2016. - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded. - Please see response No.3 - Noted - Please see response No.3 - The change in traffic behaviour will be monitored by the Roads Dept. and any undesirable behaviour will be discouraged by traffic management measures. - The provision of public transport is outside the capacity of this scheme. However this scheme is being designed to bring some measure of choice for the residents of Lucan and to encourage parents to allow their children to cycle to school rather than being driven. The proposed cycle network produced by the NTA shows a primary cycle route along the N4 to the City Centre. There is a proposed secondary route along the ORR to join with the N4 primary route, which is part of this scheme. This scheme will provide a safe route for school children to cycle to the large number of schools in Lucan village and commuters to the primary route to the City Centre along the N4. - This is outside the capacity of this scheme. |
|  | Submission by Resident (23), (25), (150), (167), (175), (177) |  |
| 10. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. |
| 10. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded. |
|  | Submission by Resident (26) |  |
| 11. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Notes the high volume of traffic travelling through Lucan Village currently - Proposes a review of traffic layout and parking in Lucan Village. - Current cycle lanes facilitates are adequate due to low numbers of cyclists in the area. |
| 11. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded. - Noted - This is outside the capacity of this scheme. - This scheme is being designed to bring some measure of choice for the residents of Lucan and to encourage parents to allow their children to cycle to school rather than being driven. The proposed cycle network produced by the NTA shows a primary cycle route along the N4 to the City Centre. There is a proposed secondary route along the ORR to join with the N4 primary route, which is part of this scheme. This scheme will provide a safe route for school children to cycle to the large number of schools in Lucan village and commuters to the primary route to the City Centre along the N4. Whilst cycle numbers may not be significant at present, providing high quality cycle facilities will help to increase cycle figures in the area. This has already been shown to be the case, the recent CSO figures has shown an increase in cycling of 43% between 2011 and 2016. |
|  | Submission by Resident (28) |  |
| 12. | Submission | - Notes the high volume of traffic travelling through Lucan Village currently - People use Lucan Village as a route to avoiding M50 Toll which adds to congestion. - Removal of right turn will significantly increase traffic volumes driving through Lucan Village to gain access to N4 at Newcastle Road (Junction 4) N4/R120 - How will this affect emergency vehicle access? - Plans should be to encourage walking to school and work. - Lucan area need had a better public transport system. |
| 12. | Response | - Noted - Noted - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded. - During emergency incidents only, emergency vehicles will still be able to perform this manoeuvre - This scheme is being designed to bring some measure of choice for the residents of Lucan and to encourage parents to allow their children to cycle to school rather than being driven. The proposed cycle network produced by the NTA shows a primary cycle route along the N4 to the City Centre. There is a proposed secondary route along the ORR to join with the N4 primary route, which is part of this scheme. This scheme will provide a safe route for school children to cycle to the large number of schools in Lucan village and commuters to the primary route to the City Centre along the N4. Whilst cycle numbers may not be significant at present, providing high quality cycle facilities will help to increase cycle figures in the area. This has already been shown to be the case, the recent CSO figures has shown an increase in cycling of 43% between 2011 and 2016. by traffic management measures i.e. banned turns etc. - The provision of public transport is outside the capacity of this scheme. |
|  | Submission by Resident (31) |  |
| 13. | Submission | - Expresses concerns about proposed works to be carries out under “Phase 2 of N4 City Centre cycle and Pedestrian Improvements Scheme” - Removal of right turn will significantly increase traffic volumes driving through Lucan Village. - People use Lucan Village as a route to avoiding M50 Toll which adds to congestion. - How will traffic access N4 Westbound? - Objects to the setting back of the existing bus stop on the south side of Old Lucan Rd - The Bus which currently blocks traffic helps people exit Esker Lane - Regular accidents at the Esker Lane junction. |
| 13. | Response | - Noted - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded. - Noted - The N4 Westbound traffic will either travel through Lucan Village to the Newcastle interchange or else travel through Willsbrook Rd to this interchange. - Noted, the bus stop is being set back to prevent buses from blocking traffic through the R136/Old Lucan Rd junction. There will be a yellow box incorporated as part of the scheme, which will help residents to exit the junction at Esker Lane. - Please see response No. 5 - Noted |
|  | Submission by Resident (32) |  |
| 14. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Negatively impact on business in Lucan Retail Park. - Notes the high volume of traffic travelling through Lucan Village currently. - Removal of right turn will significantly increase traffic volumes driving through Lucan Village. - How will traffic access N4 Westbound? - Removing the right turn is counter intuitive because the junction will be a major interchange for the future extension of the Outer Ring Road (ORR). - Suggested alternative routes are lengthy either in distance or time. |
| 14. | Response | - Noted - Noted - Noted - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded. - The N4 Westbound traffic will either travel through Lucan Village to the Newcastle interchange or else travel through Willsbrook Rd to this interchange. - There is no proposed extension of the ORR. - We believe that the additional time taken to divert will be 5 min. |
|  | Submission by Resident (34) |  |
| 15. | Submission | - Objects to proposed works to be carries out under “Phase 2 of N4 City Centre cycle and Pedestrian Improvements Scheme” to the area in and around Woodville Estate. - Putting cycle lane and bus bay in front of Woodville estate will negativity effect the look of the area. - The proposed cycle lane is to finish at Esker lane will it be extended to Lucan Village in future. - Objects to the removal of trees to add a set down bus stop in front of Woodville Downs’s estate. - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Notes the high volume of traffic travelling through Lucan Village currently. |
| 15. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded - Noted, the bus stop is being set back to prevent buses from blocking traffic through the R136/Old Lucan Rd junction. There will be a yellow box incorporated as part of the scheme, which will help residents to exit the junction at Esker Lane - Yes it will be continued as it is part of a cycle route on the National Transport Authority Greater Dublin Cycle Network. - Please see response No. 2 - Please see response No. 1 - Noted |
|  | Submission by Resident (39) |  |
| 16. | Submission | - This Scheme allow for the separation of buses and cyclists, thereby increasing safety for cyclists. - How will traffic access N4 Westbound? |
| 16. | Response | - Noted - The N4 Westbound traffic will either travel through Lucan Village to the Newcastle interchange or else travel through Willsbrook Rd to this interchange. |
|  | Submission by Resident (41) |  |
| 17. | Submission | This Submission relates to the following issues: - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Removal of right turn will block access to the N4 for residents of Lucan. - Removal of right turn will force traffic through to Liffey Valley N4 Junction in order to gain access to the N4 causing increased travel times. - Believes that there will be major traffic disruptions. The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location during construction of the scheme. - Submission outlines existing cycle lane on the R136 are rarely used. - Objects to the proposed removal of the left turning Filter Lane which runs along R136 (ORR) (Northbound) towards Lucan. |
| 17. | Response | - Noted - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded - Please see response No. 2 - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location. During peak hour, only two way traffic management systems will be allowed, thus keeping traffic disruption to a minimum. - This scheme is being designed to bring some measure of choice for the residents of Lucan and to encourage parents to allow their children to cycle to school rather than being driven. The proposed cycle network produced by the NTA shows a primary cycle route along the N4 to the City Centre. There is a proposed secondary route along the ORR to join with the N4 primary route, which is part of this scheme. This scheme will provide a safe route for school children to cycle to the large number of schools in Lucan village and commuters to the primary route to the City Centre along the N4. Whilst cycle numbers may not be significant at present, providing high quality cycle facilities will help to increase cycle figures in the area. This has already been shown to be the case, the recent CSO figures has shown an increase in cycling of 43% between 2011 and 2016. - The removal of the filter lane is designed to improve facilities for pedestrians. |
|  | Submission by Resident (43) |  |
| 18. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Objects to the removal of trees to add a set down bus stop in front of Woodville Downs estate. - Objects to the setting back of the existing bus stop on the south side of Old Lucan Rd - The Bus which currently blocks traffic helps people exit Esker Lane - Notes the high volume of traffic travelling through Lucan Village currently - People use Lucan Village as a route to avoiding M50 Toll which adds to congestion. |
| 18. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded. - Noted, the bus stop is being set back to prevent buses from blocking traffic through the R136/Old Lucan Rd junction. There will be a yellow box incorporated as part of the scheme, which will help residents to exit the junction at Esker Lane - Please see response No. 2 - Please see response No. 2 - Noted - Noted |
|  | Submission by Resident (45) |  |
| 19. | Submission | - Current cycle lanes facilitates are adequate due to low numbers of cyclists in the area. - Negatively impact on business in Lucan Retail Park. - The Proposed Cycle Lane would get a minimal amount of Cyclists due to its peripheral location on the outskirts of the village. - There is alternative route for cyclist travelling to the city along the Grand Canal Greenway. - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Raises concerns regarding traffic volumes making the right turn evening peak time. - Removal of right turn will significantly increase traffic volumes driving through Lucan Village - Removal of right turn will force traffic to join N4 at the congested Newcastle road (Junction 4) N4/R120 - Banning the right turn would cause excess traffic and lead to congestion in roads around housing estates and endanger the lives of children as it would significantly increase traffic around schools - Notes the high volume of traffic travelling through Lucan Village currently - Removal of right turn will force traffic through areas such as the already gridlocked Willsbrook Road. |
| 19. | Response | - This scheme is being designed to bring some measure of choice for the residents of Lucan and to encourage parents to allow their children to cycle to school rather than being driven. The proposed cycle network produced by the NTA shows a primary cycle route along the N4 to the City Centre. There is a proposed secondary route along the ORR to join with the N4 primary route, which is part of this scheme. This scheme will provide a safe route for school children to cycle to the large number of schools in Lucan village and commuters to the primary route to the City Centre along the N4. Whilst cycle numbers may not be significant at present, providing high quality cycle facilities will help to increase cycle figures in the area. This has already been shown to be the case, the recent CSO figures has shown an increase in cycling of 43% between 2011 and 2016. - Noted - Please see response No.1 - The Grand Canal Greenway represents a considerably detour for cyclists going to the City. - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded. - Please see response No.5 - Please see response No.5 - Please see response No.5 - Any change in traffic behaviour will be monitored by the Roads Dept. and any undesirable behaviour will be discouraged by traffic management measures - Noted - Please see response No.5 |
|  | Submission by Resident (46) |  |
| 20. | Submission | - Notes the high volume of traffic travelling through Lucan Village currently - People use Lucan Village as a route to avoiding M50 Toll which adds to congestion. - Removal of right turn will significantly increase traffic volumes driving through Lucan Village to gain access to N4 at Newcastle Road (Junction 4) N4/R120 - Health and Safety concern regarding motorist who may perform right turn, ignoring the restriction. - Health and Safety concern regarding motorist may perform U-turn at the entrance to a housing estate |
| 20. | Response | - Noted - Noted - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded. - The change in traffic behaviour will be monitored by the Roads Dept. and any undesirable behaviour will be discouraged by traffic management measures. - Please see response No.4 |
|  | Submission by Resident (48) |  |
| 21. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Removal of right turn will significantly increase traffic volumes driving through Lucan Village - Removal of right turn will significantly increase traffic volumes driving through Lucan Village - People use Lucan Village as a route to avoiding M50 Toll which adds to congestion. - Banning the right turn would cause excess traffic and lead to congestion in roads around housing estates and endanger the lives of children as it would significantly increase traffic around schools |
| 21. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded - Please see response No.1 - Please see response No.1 - Noted - The change in traffic behaviour will be monitored by the Roads Dept. and any undesirable behaviour will be discouraged by traffic management measures |
|  | Submission by Resident (49) |  |
| 22. | Submission | - Objects to proposed works to be carries out under “Phase 2 of N4 City Centre cycle and Pedestrian Improvements Scheme” - Removal of right turn will significantly increase traffic volumes driving through Lucan Village. - Removal of right turn will significantly increase traffic volumes driving through Lucan Village to gain access to N4 at Newcastle Road (Junction 4) N4/R120 |
| 22. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded - Please see response No.1 - Please see response No.1 |
|  | Submission by Resident (52) |  |
| 23. | Submission | - Objects to proposed works to be carries out under “Phase 2 of N4 City Centre cycle and Pedestrian Improvements Scheme” - Removal of right turn will force traffic through to Liffey Valley N4 Junction in order to gain access to the N4 causing increased travel times. - Removal of right turn will significantly increase traffic volumes driving through Lucan Village to gain access to N4 at Newcastle Road (Junction 4) N4/R120 - Health and Safety concern regarding motorist may perform U-turn at the entrance to Hermitage housing estate - Current cycle lanes facilitates are adequate due to low numbers of cyclists in the area. - Concerned about the safety cyclists using the newly proposed road layout for the Texaco slip road. - The proposed cycle lane is to finish at Esker lane will it be extended to Lucan Village in future. - Asks why the plans were not on public display in Lucan as part of this "Public Consultation”. - How will traffic access N4 Westbound? |
| 23. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded. - Please see response No.1 - Please see response No.1 - Any change in traffic behaviour will be monitored by the Roads Dept. and any undesirable behaviour will be discouraged by traffic management measures. - This scheme is being designed to bring some measure of choice for the residents of Lucan and to encourage parents to allow their children to cycle to school rather than being driven. The proposed cycle network produced by the NTA shows a primary cycle route along the N4 to the City Centre. There is a proposed secondary route along the ORR to join with the N4 primary route, which is part of this scheme. This scheme will provide a safe route for school children to cycle to the large number of schools in Lucan village and commuters to the primary route to the City Centre along the N4. Whilst cycle numbers may not be significant at present, providing high quality cycle facilities will help to increase cycle figures in the area. This has already been shown to be the case, the recent CSO figures has shown an increase in cycling of 43% between 2011 and 2016. - The scheme is required to go through a detailed design procedure, including a mandatory Road Safety Audit process. Due to the high speed on this slip road, consideration to changing the cycle facility to a segregated cycle facility will be investigated. It will be designed to the highest standards as prescribed in the National Cycle Manual. - Yes it will be continued as it is part of a cycle route on the National Transport Authority Greater Dublin Cycle Network - The public consultation was carried out as required, including a public display at the SDCC Offices and advertisements in the Local Newspapers. - The N4 Westbound traffic will either travel through Lucan Village to the Newcastle interchange or else travel through Willsbrook Rd to this interchange. |
|  | Submission by Resident (53) |  |
| 24. | Submission | - Objects to proposed works to be carries out under “Phase 2 of N4 City Centre cycle and Pedestrian Improvements Scheme” - Raises concerns regarding traffic volumes making the right turn evening peak time. - Health and Safety concern regarding motorist may perform U-turn at the entrance to a housing estate - Objects to the removal of trees to add a set down bus stop in front of Woodville Downs’s estate. - Cars running sitting in traffic jams will increase pollution. |
| 24. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded - Please see response No.1. - Any change in traffic behaviour will be monitored by the Roads Dept. and any undesirable behaviour will be discouraged by traffic management measures - Noted, the bus stop is being set back to prevent buses from blocking traffic through the R136/Old Lucan Rd junction. There will be a yellow box incorporated as part of the scheme, which will help residents to exit the junction at Esker Lane - This scheme is being designed to bring some measure of choice for the residents of Lucan and to encourage parents to allow their children to cycle to school rather than being driven. The proposed cycle network produced by the NTA shows a primary cycle route along the N4 to the City Centre. There is a proposed secondary route along the ORR to join with the N4 primary route, which is part of this scheme. This scheme will provide a safe route for school children to cycle to the large number of schools in Lucan village and commuters to the primary route to the City Centre along the N4. Whilst cycle numbers may not be significant at present, providing high quality cycle facilities will help to increase cycle figures in the area. This has already been shown to be the case, the recent CSO figures has shown an increase in cycling of 43% between 2011 and 2016. |
|  | Submission by Resident (54) |  |
| 25. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - How will this affect emergency vehicle access? - How will traffic access N4 Westbound? |
| 25. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded - During emergency incidents only, emergency vehicles will still be able to perform this manoeuvre - The N4 Westbound traffic will either travel through Lucan Village to the Newcastle interchange or else travel through Willsbrook Rd to this interchange. |
|  | Submission by Resident (56) |  |
| 26. | Submission | - The Submitter, as a Lucan resident, notes the high traffic congestion currently experienced in the Lucan area, - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4 - Removal of right turn will significantly increase traffic volumes driving through Lucan Village. - Local traffic is increasing all the time. |
| 26. | Response | - Noted - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded - Please see response No. 2. - Noted however this scheme is being designed to bring some measure of choice for the residents of Lucan and to encourage parents to allow their children to cycle to school rather than being driven. The proposed cycle network produced by the NTA shows a primary cycle route along the N4 to the City Centre. There is a proposed secondary route along the ORR to join with the N4 primary route, which is part of this scheme. This scheme will provide a safe route for school children to cycle to the large number of schools in Lucan village and commuters to the primary route to the City Centre along the N4. Whilst cycle numbers may not be significant at present, providing high quality cycle facilities will help to increase cycle figures in the area. This has already been shown to be the case, the recent CSO figures has shown an increase in cycling of 43% between 2011 and 2016. |
|  | Submission by Resident (57), (62) |  |
| 27. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4 |
| 27. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded |
|  | Submission by Resident (58) |  |
| 28. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Raises concerns regarding traffic volumes making the right turn evening peak time. - Removal of right turn will significantly increase traffic volumes driving through Lucan Village - Removal of right turn will force traffic to join N4 at the congested Newcastle road (Junction 4) N4/R120 - Banning the right turn would cause excess traffic and lead to congestion in roads around housing estates and endanger the lives of children as it would significantly increase traffic around schools - Notes the high volume of traffic travelling through Lucan Village currently - Removal of right turn will force traffic through areas such as the already gridlocked Willsbrook Road. - Removal of right turn will force traffic to join N4 at the congested Newcastle road (Junction 4) N4/R120. - The Proposed Cycle Lane would get a minimal amount of Cyclists due to its peripheral location on the outskirts of the village. |
| 28. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded - Please see response No.1 - Please see response No.1 - Please see response No.1 - The change in traffic behaviour will be monitored by the Roads Dept. and any undesirable behaviour will be discouraged by traffic management measures - Noted - Please see response No.1 - Please see response No.1 - Noted, this scheme is being designed to bring some measure of choice for the residents of Lucan and to encourage parents to allow their children to cycle to school rather than being driven. The proposed cycle network produced by the NTA shows a primary cycle route along the N4 to the City Centre. There is a proposed secondary route along the ORR to join with the N4 primary route, which is part of this scheme. This scheme will provide a safe route for school children to cycle to the large number of schools in Lucan village and commuters to the primary route to the City Centre along the N4. Whilst cycle numbers may not be significant at present, providing high quality cycle facilities will help to increase cycle figures in the area. This has already been shown to be the case, the recent CSO figures has shown an increase in cycling of 43% between 2011 and 2016. |
|  | Submission by Resident (63) |  |
| 29. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Notes the high volume of traffic travelling through Lucan Village currently - Removal of right turn will significantly increase traffic volumes driving through Lucan Village to gain access to N4 at Newcastle Road (Junction 4) N4/R120. - Removal of right turn will force traffic through to Liffey Valley N4 Junction in order to gain access to the N4 causing increased travel times. - Banning the right turn would cause excess traffic and lead to congestion in roads around housing estates and endanger the lives of children as it would significantly increase traffic around schools - Negatively impact on business in Lucan Village. - Negatively impact on business in Lucan Retail Park. - Objects to the setting back of the existing bus stop on the south side of Old Lucan Rd - The Bus which currently blocks traffic helps people exit Esker Lane - Concerned about the safety cyclists using the newly proposed road layout for the Texaco slip road. - There is safer alternative route for children cycling to school through Willsbrook Park to gain access to the village. - Objects to the removal of trees to add a set down bus stop in front of Woodville Downs’s estate. |
| 29. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded. - Noted - Please see response No.1 - Noted - The change in traffic behaviour will be monitored by the Roads Dept. and any undesirable behaviour will be discouraged by traffic management measures - Noted - Noted - Noted, the bus stop is being set back to prevent buses from blocking traffic through the R136/Old Lucan Rd junction. There will be a yellow box incorporated as part of the scheme, which will help residents to exit the junction at Esker Lane. - Please see response No.8 - The scheme is required to go through a detailed design procedure, including a mandatory Road Safety Audit process. Due to the high speed on this slip road, consideration to changing the cycle facility to a segregated cycle facility will be investigated. It will be designed to the highest standards as prescribed in the National Cycle Manual. The scheme is required to go through a detailed design procedure, including a mandatory Road Safety Audit process. Due to the high speed on this slip road, consideration to changing the cycle facility to a segregated cycle facility will be investigated. It will be designed to the highest standards as prescribed in the National Cycle Manual. - While the Willsbrook Park cycle route is a good alternative for schoolchildren, the R136 route is a more direct route, which will link to the N4 cycle route and on to the City. - Please see response No.8 |
|  | Submission by Resident (64) |  |
| 30. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - How will traffic access N4 Westbound? - Removal of right turn will significantly increase traffic volumes driving through Lucan Village to gain access to N4 at Newcastle Road (Junction 4) N4/R120 - Notes the high volume of traffic travelling through Lucan Village currently |
| 30. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded. - The N4 Westbound traffic will either travel through Lucan Village to the Newcastle interchange or else travel through Willsbrook Rd to this interchange. - Please see response No.1 - Noted |
|  | Submission by Resident (65) |  |
| 31. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Notes the high volume of traffic travelling through Lucan Village currently - Removal of right turn will significantly increase traffic volumes driving through Lucan Village |
| 31. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded. - Noted - Please see response No.1 |
|  | Submission by Resident (66) |  |
| 32. | Submission | - Objects to proposed works to be carries out under “Phase 2 of N4 City Centre cycle and Pedestrian Improvements Scheme” - Objects to the setting back of the existing bus stop on the south side of Old Lucan Rd - The Bus which currently blocks traffic helps people exit Esker Lane - Notes the high volume of traffic travelling through Lucan Village currently - How will traffic access N4 Westbound? - Removal of right turn will significantly increase traffic volumes driving through Lucan Village to gain access to N4 at Newcastle Road (Junction 4) N4/R120 - Putting cycle lane and bus bay in front of Woodville estate will negativity effect the look of the area. - Objects to the removal of trees to add a set down bus stop in front of Woodville Downs’s estate. - Concerned about the safety cyclists using the newly proposed road layout for the Texaco slip road. |
| 32. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded. - Noted, the bus stop is being set back to prevent buses from blocking traffic through the R136/Old Lucan Rd junction. There will be a yellow box incorporated as part of the scheme, which will help residents to exit the junction at Esker Lane. - Please see response No.2 - Noted - The N4 Westbound traffic will either travel through Lucan Village to the Newcastle interchange or else travel through Willsbrook Rd to this interchange. - Please see response No.1 - Please see response No.2 - Please see response No.2 - The scheme is required to go through a detailed design procedure, including a mandatory Road Safety Audit process. Due to the high speed on this slip road, consideration to changing the cycle facility to a segregated cycle facility will be investigated. It will be designed to the highest standards as prescribed in the National Cycle Manual. |
|  | Submission by Resident (68) |  |
| 33. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - There is alternative route for cyclist travelling to the city along the Grand Canal Greenway - Current cycle lanes facilitates are adequate. - Lucan area need had a better public transport system. - Commuter cyclists prefer to us the road carriageway and bus lanes. - Proposes advisory cycle lanes are sufficient. |
| 33. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded. - The Grand Canal Greenway represents a considerably detour for cyclists going to the City. - This scheme is being designed to bring some measure of choice for the residents of Lucan and to encourage parents to allow their children to cycle to school rather than being driven. The proposed cycle network produced by the NTA shows a primary cycle route along the N4 to the City Centre. There is a proposed secondary route along the ORR to join with the N4 primary route, which is part of this scheme. This scheme will provide a safe route for school children to cycle to the large number of schools in Lucan village and commuters to the primary route to the City Centre along the N4. Whilst cycle numbers may be not significant at present, providing high quality cycle facilities will help to increase cycle figures in the area. This has already been shown to be the case, the recent CSO figures has shown an increase in cycling of 43% between 2011 and 2016. - The provision of public transport is outside the capacity of this scheme. However this scheme is being designed to bring some measure of choice for the residents of Lucan and to encourage parents to allow their children to cycle to school rather than being driven. The proposed cycle network produced by the NTA shows a primary cycle route along the N4 to the City Centre. There is a proposed secondary route along the ORR to join with the N4 primary route, which is part of this scheme. This scheme will provide a safe route for school children to cycle to the large number of schools in Lucan village and commuters to the primary route to the City Centre along the N4. - Please see response No.1 - Please see response No.4 |
|  | Submission by Resident (70) |  |
| 34. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Notes the high volume of traffic travelling through Lucan Village currently - Local traffic is increasing all the time. - Lucan/Adamstown is a massively growing community and believes N4/R136 Junction is integral to its growth. - How will traffic access N4 Westbound? - Removal of right turn will significantly increase traffic volumes driving through Lucan Village to gain access to N4 at Newcastle Road (Junction 4) N4/R120 - Removal of right turn will block access to the N4 for residents of Lucan. - Health and Safety concern regarding motorist may perform U-turn at the entrance to a housing estate. |
| 34. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded - Noted - This scheme is being designed to bring some measure of choice for the residents of Lucan and to encourage parents to allow their children to cycle to school rather than being driven. The proposed cycle network produced by the NTA shows a primary cycle route along the N4 to the City Centre. There is a proposed secondary route along the ORR to join with the N4 primary route, which is part of this scheme. This scheme will provide a safe route for school children to cycle to the large number of schools in Lucan village and commuters to the primary route to the City Centre along the N4. Whilst cycle numbers may be not significant at present, providing high quality cycle facilities will help to increase cycle figures in the area. This has already been shown to be the case, the recent CSO figures has shown an increase in cycling of 43% between 2011 and 2016 - Noted - The N4 Westbound traffic will either travel through Lucan Village to the Newcastle interchange or else travel through Willsbrook Rd to this interchange. - Please see response No. 1. - Please see response No. 1 - The change in traffic behaviour will be monitored by the Roads Dept. and any undesirable behaviour will be discouraged by traffic management measures. |
|  | Submission by Resident (73) |  |
| 35. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Removal of right turn will block access to the N4 for residents of Lucan. - Removal of right turn will significantly increase traffic volumes driving through Lucan Village to gain access to N4 at Newcastle Road (Junction 4) N4/R120. - Notes the high volume of traffic travelling through Lucan Village currently - Removal of right turn will significantly increase traffic volumes driving through Lucan Village - Raises concerns regarding traffic volumes making the right turn evening peak time. |
| 35. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded - Please see response No. 1 - Please see response No. 1 - Noted - Please see response No. 1 - Please see response No. 1 |
|  | Submission by Resident (74) |  |
| 36. | Submission | - Notes the Junction is important for access to local schools. - Objects to the proposed removal of the left turning Filter Lane which runs along R136 (ORR) (Northbound) towards Lucan. - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - How will traffic access N4 Westbound? - Removal of right turn will significantly increase traffic volumes driving through Lucan Village |
| 36. | Response | - This scheme is being designed to bring some measure of choice for the residents of Lucan and to encourage parents to allow their children to cycle to school rather than being driven. The proposed cycle network produced by the NTA shows a primary cycle route along the N4 to the City Centre. There is a proposed secondary route along the ORR to join with the N4 primary route, which is part of this scheme. This scheme will provide a safe route for school children to cycle to the large number of schools in Lucan village and commuters to the primary route to the City Centre along the N4. Whilst cycle numbers may not be significant at present, providing high quality cycle facilities will help to increase cycle figures in the area. This has already been shown to be the case, the recent CSO figures has shown an increase in cycling of 43% between 2011 and 2016. - Noted, the removal of the filter lane is designed to improve facilities for pedestrians. - Please see response No. 1 - The N4 Westbound traffic will either travel through Lucan Village to the Newcastle interchange or else travel through Willsbrook Rd to this interchange. - Please see response No. 1 |
|  | Submission by Resident (75) |  |
| 37. | Submission | - Objects to the proposed road changes to R136 (Outer Ring Rd)/Old Lucan Rd roundabout junction. - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Notes the high volume of traffic travelling through Lucan Village currently |
| 37. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded. - Please see response No.1 - Noted |
|  | Submission by Resident (77) |  |
| 38. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Objects to the proposed removal of the left turning Filter Lane which runs along R136 (ORR) (Northbound) towards Lucan. |
| 38. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded. - The removal of the filter lane is designed to improve facilities for pedestrians. |
|  | Submission by Resident (83) |  |
| 39. | Submission | - Concerned about the safety cyclists using the newly proposed road layout for the Texaco slip road. - Removal of right turn will significantly increase traffic volumes driving through Lucan Village. |
| 39. | Response | - The scheme is required to go through a detailed design procedure, including a mandatory Road Safety Audit process. Due to the high speed on this slip road, consideration to changing the cycle facility to a segregated cycle facility will be investigated. It will be designed to the highest standards as prescribed in the National Cycle Manual. - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded. |
|  | Submission by Resident (84) |  |
| 40. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Removal of right turn will significantly increase traffic volumes driving through Lucan Village - Banning the right turn would cause excess traffic and lead to congestion in roads around housing estates and endanger the lives of children as it would significantly increase traffic around schools. - Suggests a new pedestrian bridge to the west of existing bridges. |
| 40. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded. - Please see response No. 1 - The change in traffic behaviour will be monitored by the Roads Dept. and any undesirable behaviour will be discouraged by traffic management measures - Whilst a pedestrian bridge has some merit, it does require the mixing of pedestrians and cyclists and requires to exit the bridge at a pedestrian crossing. The best design for cyclists is for them to have segregated cycle routes north and southbound over the bridge. |
|  | Submission by Resident (86) |  |
| 41. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Lucan/Adamstown is a massively growing community and believes N4/R136 Junction is integral to its growth - How will traffic access N4 Westbound? - Removal of right turn will force traffic through areas such as the already gridlocked Willsbrook Road. - Proposes the reopening of previously closed exits/entrances to N4. - Removing the right turn is counter intuitive because the junction will be a major interchange for the future extension of the Outer Ring Road (ORR). - Negatively impact on business in Lucan Retail Park. - Lucan/Adamstown is a massively growing community and believes N4/R136 Junction is integral to its growth. - Suggests a widening of the existing bridge to the west to accommodate a new footpath and cycle lane - The demand for the right turn by motor vehicles out numbers the amount of cyclists using the route. - Current cycle lanes facilitates are adequate due to low numbers of cyclists in the area. |
| 41. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded. - Noted. - The N4 Westbound traffic will either travel through Lucan Village to the Newcastle interchange or else travel through Willsbrook Rd to this interchange. - Please see response No. 1 - This is outside the capacity of this scheme - There is no proposed extension of the ORR. - Noted - Noted - This is outside the capacity of this scheme. However it is prudent and logical to try a cheaper option first before embarking on a bigger and more costly one. - Please see response No. 1. - This scheme is being designed to bring some measure of choice for the residents of Lucan and to encourage parents to allow their children to cycle to school rather than being driven. The proposed cycle network produced by the NTA shows a primary cycle route along the N4 to the City Centre. There is a proposed secondary route along the ORR to join with the N4 primary route, which is part of this scheme. This scheme will provide a safe route for school children to cycle to the large number of schools in Lucan village and commuters to the primary route to the City Centre along the N4. Whilst cycle numbers may not be significant at present, providing high quality cycle facilities will help to increase cycle figures in the area. This has already been shown to be the case, the recent CSO figures has shown an increase in cycling of 43% between 2011 and 2016 |
|  | Submission by Resident (88) |  |
| 42. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Removal of right turn will block access to the N4 for residents of Lucan. |
| 42. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded. - Please see response No. 2 |
|  | Submission by Resident (89) |  |
| 43. | Submission | - Objects to proposed works to be carries out under “Phase 2 of N4 City Centre cycle and Pedestrian Improvements Scheme” - Removal of right turn will block access to the N4 for residents of Lucan. - Removal of right turn will force traffic through to Liffey Valley N4 Junction in order to gain access to the N4 causing increased travel times. - Believes that there will be major traffic disruptions the scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location during construction of the scheme. - Objects to the proposed removal of the left turning Filter Lane which runs along R136 (ORR) (Northbound) towards Lucan. |
| 43. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded - Please see response No.1 - Please see response No.1 - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location. During peak hour, only two way traffic management systems will be allowed, thus keeping traffic disruption to a minimum - The removal of the filter lane is designed to improve facilities for pedestrians. |
|  | Submission by Resident (90) |  |
| 44. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - The Proposed Cycle Lane would get a minimal amount of Cyclists due to its peripheral location on the outskirts of the village. - Concerned about the safety cyclists using the newly proposed road layout for the Texaco slip road. - Objects to the setting back of the existing bus stop on the south side of Old Lucan Rd - The Bus which currently blocks traffic helps people exit Esker Lane - Removal of right turn will significantly increase traffic volumes driving through Lucan Village to gain access to N4 at Newcastle Road (Junction 4) N4/R120 - Health and Safety concern regarding motorist may perform U-turn at the entrance to Hermitage housing estate - The proposed cycle lane is to finish at Esker lane will it be extended to Lucan Village in future. - Putting cycle lane and bus bay in front of Woodville estate will negativity effect the look of the area. |
| 44. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded - This scheme is being designed to bring some measure of choice for the residents of Lucan and to encourage parents to allow their children to cycle to school rather than being driven. The proposed cycle network produced by the NTA shows a primary cycle route along the N4 to the City Centre. There is a proposed secondary route along the ORR to join with the N4 primary route, which is part of this scheme. This scheme will provide a safe route for school children to cycle to the large number of schools in Lucan village and commuters to the primary route to the City Centre along the N4. Whilst cycle numbers may not be significant at present, providing high quality cycle facilities will help to increase cycle figures in the area. This has already been shown to be the case, the recent CSO figures has shown an increase in cycling of 43% between 2011 and 2016. - Noted, the scheme is required to go through a detailed design procedure, including a mandatory Road Safety Audit process. Due to the high speed on this slip road, consideration to changing the cycle facility to a segregated cycle facility will be investigated. It will be designed to the highest standards as prescribed in the National Cycle Manual. - Noted, the bus stop is being set back to prevent buses from blocking traffic through the R136/Old Lucan Rd junction. There will be a yellow box incorporated as part of the scheme, which will help residents to exit the junction at Esker Lane. - Please see response No. 4 - Please see response No 1. - The change in traffic behaviour will be monitored by the Roads Dept. and any undesirable behaviour will be discouraged by traffic management measures. - Yes it will be continued as it is part of a cycle route on the National Transport Authority Greater Dublin Cycle Network - Please see response No. 4 |
|  | Submission by Resident (93) |  |
| 45. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Notes the high volume of traffic travelling through Lucan Village currently - Removal of right turn will block access to the N4 for residents of Lucan. - Current cycle lanes facilitates are adequate due to low numbers of cyclists in the area. |
| 45. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded. - Noted - Please see response No. 1 - This scheme is being designed to bring some measure of choice for the residents of Lucan and to encourage parents to allow their children to cycle to school rather than being driven. The proposed cycle network produced by the NTA shows a primary cycle route along the N4 to the City Centre. There is a proposed secondary route along the ORR to join with the N4 primary route, which is part of this scheme. This scheme will provide a safe route for school children to cycle to the large number of schools in Lucan village and commuters to the primary route to the City Centre along the N4. Whilst cycle numbers may not be significant at present, providing high quality cycle facilities will help to increase cycle figures in the area. This has already been shown to be the case, the recent CSO figures has shown an increase in cycling of 43% between 2011 and 2016 |
|  | Submission by Resident (101) |  |
| 46. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Notes the high volume of traffic travelling through Lucan Village currently - Removal of right turn will block access to the N4 for residents of Lucan. - Current cycle lanes facilitates are adequate due to low numbers of cyclists in the area. |
| 46. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded. - Noted - Please see response No. 2 - The provision of public transport is outside the capacity of this scheme. However this scheme is being designed to bring some measure of choice for the residents of Lucan and to encourage parents to allow their children to cycle to school rather than being driven. The proposed cycle network produced by the NTA shows a primary cycle route along the N4 to the City Centre. There is a proposed secondary route along the ORR to join with the N4 primary route, which is part of this scheme. This scheme will provide a safe route for school children to cycle to the large number of schools in Lucan village and commuters to the primary route to the City Centre along the N4. Whilst cycle numbers may not be significant at present, providing high quality cycle facilities will help to increase cycle figures in the area. This has already been shown to be the case, the recent CSO figures has shown an increase in cycling of 43% between 2011 and 2016. |
|  | Submission by Resident (102), (103), (104), (105), (123) |  |
| 47. | Submission | Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4 - For to the proposed extension of the right turning lane from R136 (ORR)(Southbound) on to Willsbrook Rd in order to increase capacity and prevent delays to southbound traffic - Objects to the proposed improved crossing facilities on all arms of the R136 (Outer Ring Rd)/Old Lucan Rd roundabout junction. - Objects to the proposed Bus stop improvements with Real Time Passenger Information (RPTI) and improved set-down facilities, including setting back the existing bus stop on the south side of Old Lucan Rd, which currently blocks traffic |
| 47. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded - Noted - The removal of the filter lane is designed to improve facilities for pedestrians. - Noted, the bus stop is being set back to prevent buses from blocking traffic through the R136/Old Lucan Rd junction. There will be a yellow box incorporated as part of the scheme, which will help residents to exit the junction at Esker Lane. |
|  | Submission by Resident (106), (119), (135) |  |
| 48. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4 - For to the proposed extension of the right turning lane from R136 (ORR)(Southbound) on to Willsbrook Rd in order to increase capacity and prevent delays to southbound traffic - For to the proposed improved crossing facilities on all arms of the R136 (Outer Ring Rd)/Old Lucan Rd roundabout junction. - Objects to the proposed Bus stop improvements with Real Time Passenger Information (RPTI) and improved set-down facilities, including setting back the existing bus stop on the south side of Old Lucan Rd, which currently blocks traffic. |
| 48. | Response | - This scheme is being designed to bring some measure of choice for the residents of Lucan and to encourage parents to allow their children to cycle to school rather than being driven. The proposed cycle network produced by the NTA shows a primary cycle route along the N4 to the City Centre. There is a proposed secondary route along the ORR to join with the N4 primary route, which is part of this scheme. This scheme will provide a safe route for school children to cycle to the large number of schools in Lucan village and commuters to the primary route to the City Centre along the N4. Whilst cycle numbers may not be significant at present, providing high quality cycle facilities will help to increase cycle figures in the area. This has already been shown to be the case, the recent CSO figures has shown an increase in cycling of 43% between 2011 and 2016. - Noted - Noted - Noted, the bus stop is being set back to prevent buses from blocking traffic through the R136/Old Lucan Rd junction. There will be a yellow box incorporated as part of the scheme, which will help residents to exit the junction at Esker Lane. |
|  | Submission by Resident (107), (108), (109), (113) |  |
| 49. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4 - For to the proposed extension of the right turning lane from R136 (ORR)(Southbound) on to Willsbrook Rd in order to increase capacity and prevent delays to southbound traffic - For to the proposed improved crossing facilities on all arms of the R136 (Outer Ring Rd)/Old Lucan Rd roundabout junction. |
| 49. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded. - Noted - Noted |
|  | Submission by Resident (110), (111), (112), (118), (122), (126), (132), (133), (134), (136), (138), (139), (140) |  |
| 50. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4 - For to the proposed extension of the right turning lane from R136 (ORR)(Southbound) on to Willsbrook Rd in order to increase capacity and prevent delays to southbound traffic - For to the proposed improved crossing facilities on all arms of the R136 (Outer Ring Rd)/Old Lucan Rd roundabout junction. - For to the proposed Bus stop improvements with Real Time Passenger Information (RPTI) and improved set-down facilities, including setting back the existing bus stop on the south side of Old Lucan Rd, which currently blocks traffic. |
| 50. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded. - Noted - Noted - Noted |
|  | Submission by Resident (115), (116) |  |
| 51. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4 - Objects to the proposed extension of the right turning lane from R136 (ORR)(Southbound) on to Willsbrook Rd in order to increase capacity and prevent delays to southbound traffic - For to the proposed improved crossing facilities on all arms of the R136 (Outer Ring Rd)/Old Lucan Rd roundabout junction. - For to the proposed Bus stop improvements with Real Time Passenger Information (RPTI) and improved set-down facilities, including setting back the existing bus stop on the south side of Old Lucan Rd, which currently blocks traffic |
| 51. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded. - The extension of the right turn at this location will increase the efficiency of the junction and will not have any negative impact on both vulnerable road users and vehicles. At the moment due to the inadequacy of the right turning lane right turning vehicles are blocking the straight through lane impacting on the efficient operation of the junction - Noted - Noted |
|  | Submission by Resident (117), (120), (121) |  |
| 52. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4 - For the proposed extension of the right turning lane from R136 (ORR)(Southbound) on to Willsbrook Rd in order to increase capacity and prevent delays to southbound traffic - For the proposed improved crossing facilities on all arms of the R136 (Outer Ring Rd)/Old Lucan Rd roundabout junction. - For the proposed Bus stop improvements with Real Time Passenger Information (RPTI) and improved set-down facilities, including setting back the existing bus stop on the south side of Old Lucan Rd, which currently blocks traffic. - Removal of right turn will significantly increase traffic volumes driving through Lucan Village. - Health and Safety concern regarding motorist may perform U-turn at the entrance to Hermitage housing estate . |
| 52. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded. - Noted - Noted - Noted - Please see response No.1 - The change in traffic behaviour will be monitored by the Roads Dept. and any undesirable behaviour will be discouraged |
|  | Submission by Resident (124,125,131) |  |
| 53. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4 - For the proposed extension of the right turning lane from R136 (ORR)(Southbound) on to Willsbrook Rd in order to increase capacity and prevent delays to southbound traffic - Objects to the proposed improved crossing facilities on all arms of the R136 (Outer Ring Rd)/Old Lucan Rd roundabout junction. - For the proposed Bus stop improvements with Real Time Passenger Information (RPTI) and improved set-down facilities, including setting back the existing bus stop on the south side of Old Lucan Rd, which currently blocks traffic |
| 53. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded - Noted - The crossing facilities are designed to improve crossing facilities for the significant number of pedestrians in the area. - Noted |
|  | Submission by Resident (127) |  |
| 54. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4 - For the proposed extension of the right turning lane from R136 (ORR)(Southbound) on to Willsbrook Rd in order to increase capacity and prevent delays to southbound traffic - Objects to the proposed Bus stop improvements with Real Time Passenger Information (RPTI) and improved set-down facilities, including setting back the existing bus stop on the south side of Old Lucan Rd, which currently blocks traffic |
| 54. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded. - Noted - Noted, the bus stop is being set back to prevent buses from blocking traffic through the R136/Old Lucan Rd junction. There will be a yellow box incorporated as part of the scheme, which will help residents to exit the junction at Esker Lane. |
|  | Submission by Resident (128) |  |
| 55. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4 - Objects to the proposed extension of the right turning lane from R136 (ORR)(Southbound) on to Willsbrook Rd in order to increase capacity and prevent delays to southbound traffic - For the proposed Bus stop improvements with Real Time Passenger Information (RPTI) and improved set-down facilities, including setting back the existing bus stop on the south side of Old Lucan Rd, which currently blocks traffic |
| 55. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded. - The extension of the right turning lane on to Willsbrook Road is to maximise the efficiency of the lights, in particular as the ban may require more traffic to use this turn. - Noted |
|  | Submission by Resident (129) |  |
| 56. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4 - For the proposed extension of the right turning lane from R136 (ORR)(Southbound) on to Willsbrook Rd in order to increase capacity and prevent delays to southbound traffic - Objects to the proposed improved crossing facilities on all arms of the R136 (Outer Ring Rd)/Old Lucan Rd roundabout junction. - Objects to the proposed Bus stop improvements with Real Time Passenger Information (RPTI) and improved set-down facilities, including setting back the existing bus stop on the south side of Old Lucan Rd, which currently blocks traffic - Removal of right turn will significantly increase traffic volumes driving through Lucan Village. |
| 56. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded - Noted - The crossing facilities are designed to improve crossing facilities for the significant number of pedestrians in the area - Noted, the bus stop is being set back to prevent buses from blocking traffic through the R136/Old Lucan Rd junction. There will be a yellow box incorporated as part of the scheme, which will help residents to exit the junction at Esker Lane. - Please see response No. 1. |
|  | Submission by Resident (130) |  |
| 57. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4 - For the proposed improved crossing facilities on all arms of the R136 (Outer Ring Rd)/Old Lucan Rd roundabout junction. - For the proposed Bus stop improvements with Real Time Passenger Information (RPTI) and improved set-down facilities, including setting back the existing bus stop on the south side of Old Lucan Rd, which currently blocks traffic - Proposes Traffic Lights at the entrance to Esker Estate to aid residents to exit from the estate eastbound. - Removal of right turn will significantly increase traffic volumes driving through Lucan Village. |
| 57. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded - Noted - Noted - Noted, the bus stop is being set back to prevent buses from blocking traffic through the R136/Old Lucan Rd junction. There will be a yellow box incorporated as part of the scheme, which will help residents to exit the junction at Esker Lane. The change in traffic behaviour will be monitored by the Roads Dept - Please see response No. 1 |
|  | Submission by Resident (137) |  |
| 58. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4 - Objects to the proposed extension of the right turning lane from R136 (ORR)(Southbound) on to Willsbrook Rd in order to increase capacity and prevent delays to southbound traffic - For to the proposed Bus stop improvements with Real Time Passenger Information (RPTI) and improved set-down facilities, including setting back the existing bus stop on the south side of Old Lucan Rd, which currently blocks traffic - Object to the extension of the right turning lane on the old Lucan Road. |
| 58. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded - The extension of the right turn at this location will increase the efficiency of the junction and will not have any negative impact on both vulnerable road users and vehicles. At the moment due to the inadequacy of the right turning lane right turning vehicles are blocking the straight through lane impacting on the efficient operation of the junction - Noted - Please see response No.2 |
|  | Submission by Resident (141) |  |
| 59. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Removal of right turn will significantly increase traffic volumes driving through Lucan Village. - People use Lucan Village as a route to avoiding M50 Toll which adds to congestion. - How will traffic access N4 Westbound? - Health and Safety concern regarding motorist may perform U-turn at the entrance to a housing estate. |
| 59. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded. - Please see response No. 1 - Noted - The N4 Westbound traffic will either travel through Lucan Village to the Newcastle interchange or else travel through Willsbrook Rd to this interchange - The change in traffic behaviour will be monitored by the Roads Dept. and any undesirable behaviour will be discouraged |
|  | Submission by Resident (143, 144) |  |
| 60. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Removal of right turn will significantly increase traffic volumes driving through Lucan Village. - Removal of right turn will force traffic through areas such as the already gridlocked Willsbrook Road - Suggests a widening of the existing bridge to the west to accommodate a new footpath and cycle lane |
| 60. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded. - Please see response No.1. - Please see response No.1. - This is outside the capacity of this scheme. However it is prudent and logical to try a cheaper option first before embarking on a bigger and more costly one |
|  | Submission by Resident (145) |  |
| 61. | Submission | - Health and Safety concern regarding design of scheme with regard to cyclists. |
| 61. | Response | - The change in traffic behaviour will be monitored by the Roads Dept. and any undesirable behaviour will be discouraged |
|  | Submission by Resident (146. 147) |  |
| 62. | Submission | - Removal of right turn will significantly increase traffic volumes driving through Lucan Village. - Objects to the removal of trees to add a set down bus stop in front of Woodville Downs’s estate. - Objects to the reduction of green area at the front of Woodville Lane. - The Bus which currently blocks traffic helps people exit Esker Lane - Concerned about the safety cyclists using the newly proposed road layout for the Texaco slip road. |
| 62. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded - Noted, the bus stop is being set back to prevent buses from blocking traffic through the R136/Old Lucan Rd junction. There will be a yellow box incorporated as part of the scheme, which will help residents to exit the junction at Esker Lane. - Please see response No. 2 - Please see response No. 2 - Noted, the scheme is required to go through a detailed design procedure, including a mandatory Road Safety Audit process. Due to the high speed on this slip road, consideration to changing the cycle facility to a segregated cycle facility will be investigated. It will be designed to the highest standards as prescribed in the National Cycle Manual. |
|  | Submission by Resident (148) |  |
| 63. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Removal of right turn will significantly increase traffic volumes driving through Lucan Village. - Proposes a review of traffic layout and parking in Lucan Village. - Concerned about recent changes to Chalet Gardens/R835 junction and Rose Lawn/R835 junction. - There is alternative route for the cycle lane along Ballyowen Road (L1042) to Liffey Valley |
| 63. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded - Please see response No. 1 - This is outside the capacity of this scheme. - This is outside the capacity of this scheme. - This scheme is being designed to bring some measure of choice for the residents of Lucan and to encourage parents to allow their children to cycle to school rather than being driven. The proposed cycle network produced by the NTA shows a primary cycle route along the N4 to the City Centre. There is a proposed secondary route along the ORR to join with the N4 primary route, which is part of this scheme. This scheme will provide a safe route for school children to cycle to the large number of schools in Lucan village and commuters to the primary route to the City Centre along the N4. Whilst cycle numbers may not be significant at present, providing high quality cycle facilities will help to increase cycle figures in the area. This has already been shown to be the case, the recent CSO figures has shown an increase in cycling of 43% between 2011 and 2016 |
|  | Submission by Resident (149) |  |
| 64. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - How will traffic access N4 Westbound? - Removal of right turn will significantly increase traffic volumes driving through Lucan Village. - Concerned about the safety cyclists using the newly proposed road layout for the Texaco slip road. |
| 64. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded - The N4 Westbound traffic will either travel through Lucan Village to the Newcastle interchange or else travel through Willsbrook Rd to this interchange. - Please see response No. 1. - Noted, the scheme is required to go through a detailed design procedure, including a mandatory Road Safety Audit process. Due to the high speed on this slip road, consideration to changing the cycle facility to a segregated cycle facility will be investigated. It will be designed to the highest standards as prescribed in the National Cycle Manual |
|  | Submission by Resident (153) |  |
| 65. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Removal of right turn will significantly increase traffic volumes driving through Lucan Village. - Objects to the removal of trees to add a set down bus stop in front of Woodville Downs’s estate. - Objects to the reduction of green area at the front of Woodville Lane - The Bus which currently blocks traffic helps people exit Esker Lane - Concerned about the safety cyclists using the newly proposed road layout for the Texaco slip road. - The proposed cycle lane is to finish at Esker lane will it be extended to Lucan Village in future. - Current cycle lanes facilitates are adequate due to low numbers of cyclists in the area. |
| 65. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded - Please see response No. 1. - Noted, the bus stop is being set back to prevent buses from blocking traffic through the R136/Old Lucan Rd junction. There will be a yellow box incorporated as part of the scheme, which will help residents to exit the junction at Esker Lane. - Please see response No. 3 - Please see response No. 3 - Noted, the scheme is required to go through a detailed design procedure, including a mandatory Road Safety Audit process. Due to the high speed on this slip road, consideration to changing the cycle facility to a segregated cycle facility will be investigated. It will be designed to the highest standards as prescribed in the National Cycle Manual - Yes it will be continued as it is part of a cycle route on the National Transport Authority Greater Dublin Cycle Network. - This scheme is being designed to bring some measure of choice for the residents of Lucan and to encourage parents to allow their children to cycle to school rather than being driven. The proposed cycle network produced by the NTA shows a primary cycle route along the N4 to the City Centre. There is a proposed secondary route along the R136 to join with the N4 primary route, which is part of this scheme. This scheme will provide a safe route for school children to cycle to the large number of schools in Lucan village and commuters to the primary route to the City Centre along the N4. Whilst cycle numbers may not be significant at present, providing high quality cycle facilities will help to increase cycle figures in the area. This has already been shown to be the case, the recent CSO figures has shown an increase in cycling of 43% between 2011 and 2016. |
|  | Submission by Resident (154) |  |
| 66. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Objects to the reduction of green area at the front of Woodville Lane - Objects to the removal of trees to add a set down bus stop in front of Woodville Downs’s estate. - Object to the extension of the right turning lane on the old Lucan Road. - The Bus which currently blocks traffic helps people exit Esker Lane - Concerned about the safety cyclists using the newly proposed road layout for the Texaco slip road. - The proposed cycle lane is to finish at Esker lane will it be extended to Lucan Village in future. |
| 66. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded - Noted, the bus stop is being set back to prevent buses from blocking traffic through the R136/Old Lucan Rd junction. There will be a yellow box incorporated as part of the scheme, which will help residents to exit the junction at Esker Lane. - Please see response No. 2 - The extension of the right turn at this location will increase the efficiency of the junction and will not have any negative impact on both vulnerable road users and vehicles. The extension of the right turning lane will increase the stacking length for the right turners thus decreasing the green time required for this movement. - Please see response No. 2 - Noted, the scheme is required to go through a detailed design procedure, including a mandatory Road Safety Audit process. Due to the high speed on this slip road, consideration to changing the cycle facility to a segregated cycle facility will be investigated. It will be designed to the highest standards as prescribed in the National Cycle Manual - Yes it will be continued as it is part of a cycle route on the National Transport Authority Greater Dublin Cycle Network. |
|  | Submission by Resident (155), (157) |  |
| 67. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Removal of right turn will significantly increase traffic volumes driving through Lucan Village. - Health and Safety concern regarding motorist may perform U-turn at the entrance to a housing estate. - Suggested alternative routes are lengthy either in distance or time. - The Bus which currently blocks traffic helps people exit Esker Lane - The proposed cycle lane is to finish at Esker lane will it be extended to Lucan Village in future. - Objects to the reduction of green area at the front of Woodville Lane - Concerned about the safety cyclists using the newly proposed road layout for the Texaco slip road. - There is alternative route for the cycle lane along Ballyowen Road (L1042) to Liffey Valley |
| 67. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded - Please see response No. 1 - The change in traffic behaviour will be monitored by the Roads Dept. and any undesirable behaviour will be discouraged by traffic management measures - We believe that the additional time taken to divert will be 5 min - Noted, the bus stop is being set back to prevent buses from blocking traffic through the R136/Old Lucan Rd junction. There will be a yellow box incorporated as part of the scheme, which will help residents to exit the junction at Esker Lane. - Yes it will be continued as it is part of a cycle route on the National Transport Authority Greater Dublin Cycle Network - Please see response No. 5 - The scheme is required to go through a detailed design procedure, including a mandatory Road Safety Audit process. Due to the high speed on this slip road, consideration to changing the cycle facility to a segregated cycle facility will be investigated. It will be designed to the highest standards as prescribed in the National Cycle Manual. The scheme is required to go through a detailed design procedure, including a mandatory Road Safety Audit process. Due to the high speed on this slip road, consideration to changing the cycle facility to a segregated cycle facility will be investigated. It will be designed to the highest standards as prescribed in the National Cycle Manual. - This scheme is being designed to bring some measure of choice for the residents of Lucan and to encourage parents to allow their children to cycle to school rather than being driven. The proposed cycle network produced by the NTA shows a primary cycle route along the N4 to the City Centre. There is a proposed secondary route along the ORR to join with the N4 primary route, which is part of this scheme. This scheme will provide a safe route for school children to cycle to the large number of schools in Lucan village and commuters to the primary route to the City Centre along the N4. Whilst cycle numbers may not be significant at present, providing high quality cycle facilities will help to increase cycle figures in the area. This has already been shown to be the case, the recent CSO figures has shown an increase in cycling of 43% between 2011 and 2016 |
|  | Submission by Resident (156), (162), (178), (181), (182) |  |
| 68. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Removal of right turn will significantly increase traffic volumes driving through Lucan Village. |
| 68. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded - Please see response No. 1 |
|  | Submission by Resident (158) |  |
| 69. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Removal of right turn will significantly increase traffic volumes driving through Lucan Village. - Health and Safety concern regarding motorist may perform U-turn at the entrance to a housing estate. - Removal of right turn will force traffic through to Liffey Valley N4 Junction in order to gain access to the N4 causing increased travel times - The Bus which currently blocks traffic helps people exit Esker Lane - The proposed cycle lane is to finish at Esker lane will it be extended to Lucan Village in future. - Objects to the reduction of green area at the front of Woodville Lane - Concerned about the safety cyclists using the newly proposed road layout for the Texaco slip road. - There is alternative route for the cycle lane along Ballyowen Road (L1042) to Liffey Valley |
| 69. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded - Please see response No. 1 - The change in traffic behaviour will be monitored by the Roads Dept. and any undesirable behaviour will be discouraged by traffic management measures - Please see response No. 1 - Noted, the bus stop is being set back to prevent buses from blocking traffic through the R136/Old Lucan Rd junction. There will be a yellow box incorporated as part of the scheme, which will help residents to exit the junction at Esker Lane. - Yes it will be continued as it is part of a cycle route on the National Transport Authority Greater Dublin Cycle Network - Please see response No.5 - The scheme is required to go through a detailed design procedure, including a mandatory Road Safety Audit process. Due to the high speed on this slip road, consideration to changing the cycle facility to a segregated cycle facility will be investigated. It will be designed to the highest standards as prescribed in the National Cycle Manual. The scheme is required to go through a detailed design procedure, including a mandatory Road Safety Audit process. Due to the high speed on this slip road, consideration to changing the cycle facility to a segregated cycle facility will be investigated. It will be designed to the highest standards as prescribed in the National Cycle Manual. - This scheme is being designed to bring some measure of choice for the residents of Lucan and to encourage parents to allow their children to cycle to school rather than being driven. The proposed cycle network produced by the NTA shows a primary cycle route along the N4 to the City Centre. There is a proposed secondary route along the ORR to join with the N4 primary route, which is part of this scheme. This scheme will provide a safe route for school children to cycle to the large number of schools in Lucan village and commuters to the primary route to the City Centre along the N4. Whilst cycle numbers may not be significant at present, providing high quality cycle facilities will help to increase cycle figures in the area. This has already been shown to be the case, the recent CSO figures has shown an increase in cycling of 43% between 2011 and 2016 |
|  | Submission by Resident (159) |  |
| 70. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - How will traffic access N4 Westbound? - Removal of right turn will significantly increase traffic volumes driving through Lucan Village. - Current cycle lanes facilitates are adequate due to low numbers of cyclists in the area |
| 70. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded - The N4 Westbound traffic will either travel through Lucan Village to the Newcastle interchange or else travel through Willsbrook Rd to this interchange. - Please see response No. 1 - This scheme is being designed to bring some measure of choice for the residents of Lucan and to encourage parents to allow their children to cycle to school rather than being driven. The proposed cycle network produced by the NTA shows a primary cycle route along the N4 to the City Centre. There is a proposed secondary route along the ORR to join with the N4 primary route, which is part of this scheme. This scheme will provide a safe route for school children to cycle to the large number of schools in Lucan village and commuters to the primary route to the City Centre along the N4. Whilst cycle numbers may not be significant at present, providing high quality cycle facilities will help to increase cycle figures in the area. This has already been shown to be the case, the recent CSO figures has shown an increase in cycling of 43% between 2011 and 2016. |
|  | Submission by Resident (160) |  |
| 71. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Removal of right turn will significantly increase traffic volumes driving through Lucan Village. - Health and Safety concern regarding motorist may perform U-turn at the entrance to a housing estate - Removal of right turn will force traffic through to Liffey Valley N4 Junction in order to gain access to the N4 causing increased travel times - The Bus which currently blocks traffic helps people exit Esker Lane - The proposed cycle lane is to finish at Esker lane will it be extended to Lucan Village in future. - Objects to the reduction of green area at the front of Woodville Lane - Concerned about the safety cyclists using the newly proposed road layout for the Texaco slip road. - There is alternative route for the cycle lane along Ballyowen Road (L1042) to Liffey Valley - Objects to the proposed removal of the left turning Filter Lane which runs along R136 (ORR) (Northbound) towards Lucan. |
| 71. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded - Please see response No. 1 - The change in traffic behaviour will be monitored by the Roads Dept. and any undesirable behaviour will be discouraged by traffic management measures - Please see response No. 1 - Noted, the bus stop is being set back to prevent buses from blocking traffic through the R136/Old Lucan Rd junction. There will be a yellow box incorporated as part of the scheme, which will help residents to exit the junction at Esker Lane. - Yes it will be continued as it is part of a cycle route on the National Transport Authority Greater Dublin Cycle Network - Please see response No. 5 - The scheme is required to go through a detailed design procedure, including a mandatory Road Safety Audit process. Due to the high speed on this slip road, consideration to changing the cycle facility to a segregated cycle facility will be investigated. It will be designed to the highest standards as prescribed in the National Cycle Manual. The scheme is required to go through a detailed design procedure, including a mandatory Road Safety Audit process. Due to the high speed on this slip road, consideration to changing the cycle facility to a segregated cycle facility will be investigated. It will be designed to the highest standards as prescribed in the National Cycle Manual. - This scheme is being designed to bring some measure of choice for the residents of Lucan and to encourage parents to allow their children to cycle to school rather than being driven. The proposed cycle network produced by the NTA shows a primary cycle route along the N4 to the City Centre. There is a proposed secondary route along the ORR to join with the N4 primary route, which is part of this scheme. This scheme will provide a safe route for school children to cycle to the large number of schools in Lucan village and commuters to the primary route to the City Centre along the N4. Whilst cycle numbers may not be significant at present, providing high quality cycle facilities will help to increase cycle figures in the area. This has already been shown to be the case, the recent CSO figures has shown an increase in cycling of 43% between 2011 and 2016 - The removal of the filter lane is designed to improve facilities for pedestrians. |
|  | Submission by Resident (161) |  |
| 72. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Suggested alternative routes are lengthy either in distance or time. - The Bus which currently blocks traffic helps people exit Esker Lane |
| 72. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded - We believe that the additional time taken to divert will be 5 min - Noted, the bus stop is being set back to prevent buses from blocking traffic through the R136/Old Lucan Rd junction. There will be a yellow box incorporated as part of the scheme, which will help residents to exit the junction at Esker Lane |
|  | Submission by Resident (163) |  |
| 73. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Current cycle lanes facilitates are adequate due to low numbers of cyclists in the area. - Suggests a new pedestrian bridge to the west of existing bridges. - Raises concerns regarding traffic volumes making the right turn evening peak time. - The Bus which currently blocks traffic helps people exit Esker Lane - Asks why the plans were not on public display in Lucan as part of this "Public Consultation”. |
| 73. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded - This scheme is being designed to bring some measure of choice for the residents of Lucan and to encourage parents to allow their children to cycle to school rather than being driven. The proposed cycle network produced by the NTA shows a primary cycle route along the N4 to the City Centre. There is a proposed secondary route along the ORR to join with the N4 primary route, which is part of this scheme. This scheme will provide a safe route for school children to cycle to the large number of schools in Lucan village and commuters to the primary route to the City Centre along the N4. Whilst cycle numbers may not be significant at present, providing high quality cycle facilities will help to increase cycle figures in the area. This has already been shown to be the case, the recent CSO figures has shown an increase in cycling of 43% between 2011 and 2016. - This is outside the capacity of this scheme. However it is prudent and logical to try a cheaper option first before embarking on a bigger and more costly one. - Please see response No. 1 - Noted, the bus stop is being set back to prevent buses from blocking traffic through the R136/Old Lucan Rd junction. There will be a yellow box incorporated as part of the scheme, which will help residents to exit the junction at Esker Lane. - The public consultation was carried out as required, including a public display at the SDCC Offices and advertisements in the Local Newspapers. |
|  | Submission by Resident (164) |  |
| 74. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - People use Lucan Village as a route to avoiding M50 Toll which adds to congestion. - How will traffic access N4 Westbound? - Removal of right turn will significantly increase traffic volumes driving through Lucan Village. - There is alternative route for the cycle lane along Willsbrook Park to gain access to the village. |
| 74. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded - Noted - The N4 Westbound traffic will either travel through Lucan Village to the Newcastle interchange or else travel through Willsbrook Rd to this interchange. - Please see response No. 1 - While the Willsbrook Park cycle route is a good alternative for schoolchildren, the R136 route is a more direct route, which will link to the N4 cycle route and on to the City. |
|  | Submission by Resident (165), (191), (195) |  |
| 75. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Negatively impact on business in Lucan Retail Park. - How will traffic access N4 Westbound? - Suggested alternative routes are lengthy either in distance or time. |
| 75. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded - Noted - The N4 Westbound traffic will either travel through Lucan Village to the Newcastle interchange or else travel through Willsbrook Rd to this interchange. - We believe that the additional time taken to divert will be 5 min |
|  | Submission by Resident (166) |  |
| 76. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Negatively impact on business in Lucan Retail Park. - How will traffic access N4 Westbound? - Suggested alternative routes are lengthy either in distance or time. - Health and Safety concern regarding motorist may perform U-turn at the entrance to Hermitage housing estate - Removal of right turn will significantly increase traffic volumes driving through Lucan Village. - Killenn Road/Nangor Rd Junction caused negatively impacted on businesses in the area |
| 76. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded - Noted - The N4 Westbound traffic will either travel through Lucan Village to the Newcastle interchange or else travel through Willsbrook Rd to this interchange. - We believe that the additional time taken to divert will be 5 min - The change in traffic behaviour will be monitored by the Roads Dept. and any undesirable behaviour will be discouraged by traffic management measure - Please see response No. 1 - Noted |
|  | Submission by Resident (168) |  |
| 77. | Submission | - The Proposed Cycle Lane would get a minimal amount of Cyclists due to its peripheral location on the outskirts of the village. - Removal of right turn will significantly increase traffic volumes driving through Lucan Village. - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - How will traffic access N4 Westbound? - Suggested alternative routes are lengthy either in distance or time. - Object to the extension of the right turning lane on the old Lucan Road. - The Bus which currently blocks traffic helps people exit Esker Lane - Objects to the removal of trees to add a set down bus stop in front of Woodville Downs’s estate. - The proposed cycle lane is to finish at Esker lane will it be extended to Lucan Village in future. - Concerned about the safety cyclists using the newly proposed road layout for the Texaco slip road. - Asks why the plans were not on public display in Lucan as part of this "Public Consultation”. |
| 77. | Response | - This scheme is being designed to bring some measure of choice for the residents of Lucan and to encourage parents to allow their children to cycle to school rather than being driven. The proposed cycle network produced by the NTA shows a primary cycle route along the N4 to the City Centre. There is a proposed secondary route along the ORR to join with the N4 primary route, which is part of this scheme. This scheme will provide a safe route for school children to cycle to the large number of schools in Lucan village and commuters to the primary route to the City Centre along the N4. Whilst cycle numbers may not be significant at present, providing high quality cycle facilities will help to increase cycle figures in the area. This has already been shown to be the case, the recent CSO figures has shown an increase in cycling of 43% between 2011 and 2016. - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded - Please see response No. 2 - The N4 Westbound traffic will either travel through Lucan Village to the Newcastle interchange or else travel through Willsbrook Rd to this interchange. - We believe that the additional time taken to divert will be 5 min - The extension of the right turn at this location will increase the efficiency of the junction and will not have any negative impact on both vulnerable road users and vehicles. The extension of the right turning lane will increase the stacking length for the right turners thus decreasing the green time required for this movement. - Noted, the bus stop is being set back to prevent buses from blocking traffic through the R136/Old Lucan Rd junction. There will be a yellow box incorporated as part of the scheme, which will help residents to exit the junction at Esker Lane - Please see response No. 7. - Yes it will be continued as it is part of a cycle route on the National Transport Authority Greater Dublin Cycle Network - The scheme is required to go through a detailed design procedure, including a mandatory Road Safety Audit process. Due to the high speed on this slip road, consideration to changing the cycle facility to a segregated cycle facility will be investigated. It will be designed to the highest standards as prescribed in the National Cycle Manual. The scheme is required to go through a detailed design procedure, including a mandatory Road Safety Audit process. Due to the high speed on this slip road, consideration to changing the cycle facility to a segregated cycle facility will be investigated. It will be designed to the highest standards as prescribed in the National Cycle Manual. - The public consultation was carried out as required, including a public display at the SDCC Offices and advertisements in the Local Newspapers |
|  | Submission by Resident (169) |  |
| 78. | Submission | - Objects to the proposed removal of the left turning Filter Lane which runs along R136 (ORR) (Northbound) towards Lucan. |
| 78. | Response | - The removal of the filter lane is designed to improve facilities for pedestrians. |
|  | Submission by Resident (170), (171), (173), (174), (179) |  |
| 79. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Removal of right turn will significantly increase traffic volumes driving through Lucan Village. - Health and Safety concern regarding motorist may perform U-turn at the entrance to a housing estate. - Removal of right turn will force traffic through to Liffey Valley N4 Junction in order to gain access to the N4 causing increased travel times - The Bus which currently blocks traffic helps people exit Esker Lane - The proposed cycle lane is to finish at Esker lane will it be extended to Lucan Village in future. - Objects to the reduction of green area at the front of Woodville Lane - Concerned about the safety cyclists using the newly proposed road layout for the Texaco slip road. - There is alternative route for the cycle lane along Ballyowen Road (L1042) to Liffey Valley - Object to the extension of the right turning lane on the old Lucan Road. |
| 79. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded. - Please see response No. 1 - The change in traffic behaviour will be monitored by the Roads Dept. and any undesirable behaviour will be discouraged by traffic management measures. - Please see response No. 1 - Noted, the bus stop is being set back to prevent buses from blocking traffic through the R136/Old Lucan Rd junction. There will be a yellow box incorporated as part of the scheme, which will help residents to exit the junction at Esker Lane - Yes it will be continued as it is part of a cycle route on the National Transport Authority Greater Dublin Cycle Network. - Please see response No. 5 - The scheme is required to go through a detailed design procedure, including a mandatory Road Safety Audit process. Due to the high speed on this slip road, consideration to changing the cycle facility to a segregated cycle facility will be investigated. It will be designed to the highest standards as prescribed in the National Cycle Manual. The scheme is required to go through a detailed design procedure, including a mandatory Road Safety Audit process. Due to the high speed on this slip road, consideration to changing the cycle facility to a segregated cycle facility will be investigated. It will be designed to the highest standards as prescribed in the National Cycle Manual. - This scheme is being designed to bring some measure of choice for the residents of Lucan and to encourage parents to allow their children to cycle to school rather than being driven. The proposed cycle network produced by the NTA shows a primary cycle route along the N4 to the City Centre. There is a proposed secondary route along the ORR to join with the N4 primary route, which is part of this scheme. This scheme will provide a safe route for school children to cycle to the large number of schools in Lucan village and commuters to the primary route to the City Centre along the N4. Whilst cycle numbers may not be significant at present, providing high quality cycle facilities will help to increase cycle figures in the area. This has already been shown to be the case, the recent CSO figures has shown an increase in cycling of 43% between 2011 and 2016 - The extension of the right turn at this location will increase the efficiency of the junction and will not have any negative impact on both vulnerable road users and vehicles. The extension of the right turning lane will increase the stacking length for the right turners thus decreasing the green time required for this movement. |
|  | Submission by Resident (172) |  |
| 80. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Removal of right turn will significantly increase traffic volumes driving through Lucan Village. - Lucan area need had a better public transport system, cars are the only way to reliably get around Lucan and its environs. - Removal of right turn will force traffic to join N4 at the congested Newcastle road (Junction 4) N4/R120 |
| 80. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded. - Please see response No. 1 - This scheme is being designed to bring some measure of choice for the residents of Lucan and to encourage parents to allow their children to cycle to school rather than being driven. The proposed cycle network produced by the NTA shows a primary cycle route along the N4 to the City Centre. There is a proposed secondary route along the ORR to join with the N4 primary route, which is part of this scheme. This scheme will provide a safe route for school children to cycle to the large number of schools in Lucan village and commuters to the primary route to the City Centre along the N4. Whilst cycle numbers may not be significant at present, providing high quality cycle facilities will help to increase cycle figures in the area. This has already been shown to be the case, the recent CSO figures has shown an increase in cycling of 43% between 2011 and 2016 - Please see response No.1 |
|  | Submission by Resident (176) |  |
| 81. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Removal of right turn will significantly increase traffic volumes driving through Lucan Village. - Removal of right turn will block access to the N4 for residents of Lucan. |
| 81. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded. - Please see response No. 1 - Please see response No. 1 |
|  | Submission by Resident (180) |  |
| 82. | Submission | - Welcomed the changes to the pedestrian lights as the present system is slow and cumbersome. - Objects to the removal of trees to add a set down bus stop in front of Woodville Downs’s estate. - The Bus which currently blocks traffic helps people exit Esker Lane - Object to the extension of the right turning lane on the old Lucan Road. - Concerned about the safety cyclists using the newly proposed road layout for the Texaco slip road. - The proposed cycle lane is to finish at Esker lane will it be extended to Lucan Village in future. - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Health and Safety concern regarding motorist may perform U-turn at the entrance to a housing estate - Removal of right turn will force traffic through to Liffey Valley N4 Junction in order to gain access to the N4 causing increased travel times - How will traffic access N4 Westbound? - Removal of right turn will significantly increase traffic volumes driving through Lucan Village. - Asks why the plans were not on public display in Lucan as part of this "Public Consultation”. |
| 82. | Response | - Noted - Noted, the bus stop is being set back to prevent buses from blocking traffic through the R136/Old Lucan Rd junction. There will be a yellow box incorporated as part of the scheme, which will help residents to exit the junction at Esker Lane. - Please see response No. 1 - The extension of the right turn at this location will increase the efficiency of the junction and will not have any negative impact on both vulnerable road users and vehicles. The extension of the right turning lane will increase the stacking length for the right turners thus decreasing the green time required for this movement. - The scheme is required to go through a detailed design procedure, including a mandatory Road Safety Audit process. Due to the high speed on this slip road, consideration to changing the cycle facility to a segregated cycle facility will be investigated. It will be designed to the highest standards as prescribed in the National Cycle Manual. The scheme is required to go through a detailed design procedure, including a mandatory Road Safety Audit process. Due to the high speed on this slip road, consideration to changing the cycle facility to a segregated cycle facility will be investigated. It will be designed to the highest standards as prescribed in the National Cycle Manual. - Yes it will be continued as it is part of a cycle route on the National Transport Authority Greater Dublin Cycle Network - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded. - The change in traffic behaviour will be monitored by the Roads Dept. and any undesirable behaviour will be discouraged by traffic management measures. - Please see response No. 7 - The N4 Westbound traffic will either travel through Lucan Village to the Newcastle interchange or else travel through Willsbrook Rd to this interchange. - Please see response No. 7 - The public consultation was carried out as required, including a public display at the SDCC Offices and advertisements in the Local Newspapers |
|  | Submission by Resident (183) |  |
| 83. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - How was the design process timeline to choose the final road layout? - Removal of right turn will significantly increase traffic volumes driving through Lucan Village - How will traffic access N4 Westbound? - Concerned about recent changes to R835/Millstream Road Junction - Lucan/Adamstown is a massively growing community and believes N4/R136 Junction is integral to its growth. - Suggests a new pedestrian bridge to the west of existing bridges. - Removing the right turn is counter intuitive because the junction will be a major interchange for the future extension of the Outer Ring Road (ORR). - Proposals as shown, show no link from existing cycle lane on N4 inbound off-ramp to the proposed new cycle lane. - There is alternative route for the cycle lane along Willsbrook Park to gain access to the village - Proposes the reopening of previously closed exits/entrances to N4 - Proposes to remove the bus lane over the bridge to provide room for the upgrade in facilities. - Proposes a review of traffic layout and parking in Lucan Village. |
| 83. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded. - The scheme went through a design process which included option selection based on cost, safety and impact on other road users. - Please see response No. 1 - The N4 Westbound traffic will either travel through Lucan Village to the Newcastle interchange or else travel through Willsbrook Rd to this interchange. - This is outside the capacity of this scheme. - Noted - This is outside the capacity of this scheme. However it is prudent and logical to try a cheaper option first before embarking on a bigger and more costly one. - here is no proposed extension of the ORR - There will be a link to current N4 cycle lanes. - While the Willsbrook Park cycle route is a good alternative for schoolchildren, the R136 route is a more direct route, which will link to the N4 cycle route and on to the City. - This is outside the capacity of this scheme - The bus forms an integral part of the sustainable transport network in the Lucan area. This scheme is being designed to bring some measure of choice for the residents of Lucan and to encourage parents to allow their children to cycle to school rather than being driven. The proposed cycle network produced by the NTA shows a primary cycle route along the N4 to the City Centre. There is a proposed secondary route along the ORR to join with the N4 primary route, which is part of this scheme. This scheme will provide a safe route for school children to cycle to the large number of schools in Lucan village and commuters to the primary route to the City Centre along the N4. Whilst cycle numbers may not be significant at present, providing high quality cycle facilities will help to increase cycle figures in the area. This has already been shown to be the case, the recent CSO figures has shown an increase in cycling of 43% between 2011 and 2016 - This is outside the capacity of this scheme. |
|  | Submission by Resident (186) |  |
| 84. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Removal of right turn will significantly increase traffic volumes driving through Lucan Village. - Concerned about the safety cyclists using the newly proposed road layout for the Texaco slip road. - Asks why the plans were not on public display in Lucan as part of this "Public Consultation”. |
| 84. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded. - Please see response No. 1 - The scheme is required to go through a detailed design procedure, including a mandatory Road Safety Audit process. Due to the high speed on this slip road, consideration to changing the cycle facility to a segregated cycle facility will be investigated. It will be designed to the highest standards as prescribed in the National Cycle Manual. The scheme is required to go through a detailed design procedure, including a mandatory Road Safety Audit process. Due to the high speed on this slip road, consideration to changing the cycle facility to a segregated cycle facility will be investigated. It will be designed to the highest standards as prescribed in the National Cycle Manual. - The public consultation was carried out as required, including a public display at the SDCC Offices and advertisements in the Local Newspapers |
|  | Submission by Resident (187) |  |
| 85. | Submission | - Putting cycle lane and bus bay in front of Woodville estate will negativity effect the look of the area - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - How will traffic access N4 Westbound? - Health and Safety concern regarding motorist may perform U-turn at the entrance to a housing estate. - Removal of right turn will significantly increase traffic volumes driving through Lucan Village. - Removal of right turn will force traffic through to Liffey Valley N4 Junction in order to gain access to the N4 causing increased travel times - The proposed cycle lane is to finish at Esker lane will it be extended to Lucan Village in future. - Objects to the removal of trees to add a set down bus stop in front of Woodville Downs’s estate. - Objects to the reduction of green area at the front of Woodville Lane |
| 85. | Response | - Noted, the bus stop is being set back to prevent buses from blocking traffic through the R136/Old Lucan Rd junction. There will be a yellow box incorporated as part of the scheme, which will help residents to exit the junction at Esker Lane. - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded - The N4 Westbound traffic will either travel through Lucan Village to the Newcastle interchange or else travel through Willsbrook Rd to this interchange. - The change in traffic behaviour will be monitored by the Roads Dept. and any undesirable behaviour will be discouraged by traffic management measures. - Please see response No.2 - Please see response No.2 - Yes it will be continued as it is part of a cycle route on the National Transport Authority Greater Dublin Cycle Network. - Please see response No.1 - Please see response No.1 |
|  | Submission by Resident (188) |  |
| 86. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Suggested alternative routes are lengthy either in distance or time. - Recessing of the existing bus stop into the green area at the front of Woodville Downs on the old Lucan Road will not increase traffic flow. - Objects to the reduction of green area at the front of Woodville Lane - Cycle lanes should be incorporated into the existing road network and bus lanes infrastructure. |
| 86. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded. - We believe that the additional time taken to divert will be 5 min - Noted, the bus stop is being set back to prevent buses from blocking traffic through the R136/Old Lucan Rd junction. There will be a yellow box incorporated as part of the scheme, which will help residents to exit the junction at Esker Lane. - Please see response No. 1 - This scheme is being designed to bring some measure of choice for the residents of Lucan and to encourage parents to allow their children to cycle to school rather than being driven. The proposed cycle network produced by the NTA shows a primary cycle route along the N4 to the City Centre. There is a proposed secondary route along the ORR to join with the N4 primary route, which is part of this scheme. This scheme will provide a safe route for school children to cycle to the large number of schools in Lucan village and commuters to the primary route to the City Centre along the N4. Whilst cycle numbers may not be significant at present, providing high quality cycle facilities will help to increase cycle figures in the area. This has already been shown to be the case, the recent CSO figures has shown an increase in cycling of 43% between 2011 and 2016 |
|  | Submission by Resident (189) |  |
| 87. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Removal of right turn will significantly increase traffic volumes driving through Lucan Village. - Suggested alternative routes are lengthy either in distance or time. - How will traffic access N4 Westbound? - After scheme is completed submitter believes that it still will not be safe to allow a child to cycle to school. - The proposed cycle lane is to finish at Esker lane will it be extended to Lucan Village in future? - Objects to the removal of trees to add a set down bus stop in front of Woodville Downs’s estate. - There is safer alternative route for children cycling to school through Willsbrook Park to gain access to the village |
| 87. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded. - Please see response No. 1 - We believe that the additional time taken to divert will be 5 min - The N4 Westbound traffic will either travel through Lucan Village to the Newcastle interchange or else travel through Willsbrook Rd to this interchange. - This scheme is being designed to bring some measure of choice for the residents of Lucan and to encourage parents to allow their children to cycle to school rather than being driven. The proposed cycle network produced by the NTA shows a primary cycle route along the N4 to the City Centre. There is a proposed secondary route along the ORR to join with the N4 primary route, which is part of this scheme. This scheme will provide a safe route for school children to cycle to the large number of schools in Lucan village and commuters to the primary route to the City Centre along the N4. Whilst cycle numbers may not be significant at present, providing high quality cycle facilities will help to increase cycle figures in the area. This has already been shown to be the case, the recent CSO figures has shown an increase in cycling of 43% between 2011 and 2016. - Yes it will be continued as it is part of a cycle route on the National Transport Authority Greater Dublin Cycle Network. - Noted, the bus stop is being set back to prevent buses from blocking traffic through the R136/Old Lucan Rd junction. There will be a yellow box incorporated as part of the scheme, which will help residents to exit the junction at Esker Lane. - While the Willsbrook Park cycle route is a good alternative for schoolchildren, the R136 route is a more direct route, which will link to the N4 cycle route and on to the City. |
|  | Submission by Resident (190) |  |
| 88. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Removal of right turn will significantly increase traffic volumes driving through Lucan Village. - Cyclists should share the bus lane as they do currently. - Suggested alternative routes are lengthy either in distance or time. |
| 88. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded. - Please see response No.1 - Please see response No.1 - We believe that the additional time taken to divert will be 5 min |
|  | Submission by Resident (192) |  |
| 89. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - How will traffic access N4 Westbound? |
| 89. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded. - The N4 Westbound traffic will either travel through Lucan Village to the Newcastle interchange or else travel through Willsbrook Rd to this interchange. |
|  | Submission by Resident (193) |  |
| 90. | Submission | - Happy SDCC are extending the network of cycle lanes in the area to encourage the use of cycles by school goers and commuters. - The roads involved are too narrow to allow any further dedicated lanes for any purpose and any further provision of cycle lanes can only be at the expense of bus lanes, other traffic and pedestrians. - Notes the high volume of traffic travelling through Lucan Village currently - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Removal of right turn will significantly increase traffic volumes driving through Lucan Village. - Suggested alternative routes are lengthy either in distance or time. - How will traffic access N4 Westbound? - People use Lucan Village as a route to avoiding M50 Toll which adds to congestion. - Removal of right turn will force traffic through areas such as the already gridlocked Willsbrook Road. - Works at Willsbrook junction on the R136 did nothing to ease traffic congestion. - Lengthening the right-hand lane from the R136 onto Willsbrook Road is a good idea. - Proposes a review of traffic layout and parking in Lucan Village. - Object to the construction of a bus bay in front of the Woodville Estate. - The Bus which currently blocks traffic helps people exit Esker Lane - Proposes Traffic Lights at the entrance to Ballydowd Grove to aid residents to exit from the estate eastbound. - Proper facilities can’t be made without widening the Old Lucan Road which would involve purchasing land to the north of the road. - Suggests a new pedestrian bridge to the west of existing bridges. |
| 90. | Response | - Noted - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded - Noted - Please see response No. 2 - Please see response No. 2 - We believe that the additional time taken to divert will be 5 min - The N4 Westbound traffic will either travel through Lucan Village to the Newcastle interchange or else travel through Willsbrook Rd to this interchange. - Noted - This scheme is being designed to bring some measure of choice for the residents of Lucan and to encourage parents to allow their children to cycle to school rather than being driven. The proposed cycle network produced by the NTA shows a primary cycle route along the N4 to the City Centre. There is a proposed secondary route along the ORR to join with the N4 primary route, which is part of this scheme. This scheme will provide a safe route for school children to cycle to the large number of schools in Lucan village and commuters to the primary route to the City Centre along the N4. Whilst cycle numbers may not be significant at present, providing high quality cycle facilities will help to increase cycle figures in the area. This has already been shown to be the case, the recent CSO figures has shown an increase in cycling of 43% between 2011 and 2016. - Please see response No. 9 - Noted - This is outside the capacity of this scheme. - Please see response No. 2 - Noted, the bus stop is being set back to prevent buses from blocking traffic through the R136/Old Lucan Rd junction. There will be a yellow box incorporated as part of the scheme, which will help residents to exit the junction at Esker Lane. - The change in traffic behaviour will be monitored by the Roads Dept and any undesirable behaviour will be discouraged by traffic management measures. - Noted - This is outside the capacity of this scheme. However it is prudent and logical to try a cheaper option first before embarking on a bigger and more costly one. |
|  | Submission by Resident (194) |  |
| 91. | Submission | - Objects to the proposed removal of the right turn from R136 (ORR) (Southbound) onto N4 (Westbound) in Lucan in order to provide a segregated cycle route in both direction across the bridge over N4. - Concerned about the safety cyclists using the newly proposed road layout for the Texaco slip road. - There is safer alternative route for cyclists through Willsbrook Park to gain access to the village. - Objects to the removal of trees to add a set down bus stop in front of Woodville Downs’s estate. - Object to the construction of a bus bay in front of the Woodville Estate. - The Bus which currently blocks traffic helps people exit Esker Lane - Object to the extension of the right turning lane on the old Lucan Road. - Welcomed the changes to the pedestrian lights as the present system is slow and cumbersome. |
| 91. | Response | - The scheme proposes to remove the right turning lane (Westbound) in order to allow for a segregated cycle lane in both directions on the R136. We would be concerned about northbound cyclists sharing the bus lane with buses and taxis at this location due to the number of buses using this section of bus lane and due to the narrowness of the bridge. Where cyclists share bus lanes in other locations, there is generally a footpath. At this particular location, the cyclist could be crushed between a bus and the bridge parapet. In addition southbound cyclists will have a segregated cycle facility. During the morning peak there are only 29 vehicles using this right turn on to the N4 Westbound. This represents only 1.6% of all traffic crossing over the R136 (ORR) /M50 Bridge. We feel the benefits of the scheme considerably outweigh the small number of movements, which will be discommoded - The scheme is required to go through a detailed design procedure, including a mandatory Road Safety Audit process. Due to the high speed on this slip road, consideration to changing the cycle facility to a segregated cycle facility will be investigated. It will be designed to the highest standards as prescribed in the National Cycle Manual. The scheme is required to go through a detailed design procedure, including a mandatory Road Safety Audit process. Due to the high speed on this slip road, consideration to changing the cycle facility to a segregated cycle facility will be investigated. It will be designed to the highest standards as prescribed in the National Cycle Manual. - While the Willsbrook Park cycle route is a good alternative for schoolchildren, the R136 route is a more direct route, which will link to the N4 cycle route and on to the City. - Noted, the bus stop is being set back to prevent buses from blocking traffic through the R136/Old Lucan Rd junction. There will be a yellow box incorporated as part of the scheme, which will help residents to exit the junction at Esker Lane. - Please see response No.4 - Please see response No.4 - The extension of the right turn at this location will increase the efficiency of the junction and will not have any negative impact on both vulnerable road users and vehicles. The extension of the right turning lane will increase the stacking length for the right turners thus decreasing the green time required for this movement. - Noted. |

LIST OF RECOMMENDATIONS

- Due to the significant number of objections, particularly focused on the right turning ban, SDCC are proposing to construct the full scheme but introduce the ban on a trial basis on the bridge only. This will consist of flexi kerbs and bollards, to delineate the north and southbound cycle track on the bridge, and amended lane markings. The rest of the cycle scheme will be permanent works. A survey including queue lengths will be carried out before the works commence and six months after to gauge the effect of the ban. If the ban is found to be detrimental to the traffic movements in and around this location, other options will be explored.  In addition, cycle training workshops will be set up in all the local schools in order to encourage cycling and to publicise the cycle tracks in the area.

LIST OF SUBMISSIONS

| No. | Name | Address |  |  |
| --- | --- | --- | --- | --- |
| 1 | Catherine Kinsella | None given | None given | None given |
| 2 | Shane Callan | None given | None given | None given |
| 3 | Jenny Begley | 30 Ballyowen Avenue | Lucan | Co. Dublin |
| 4 | Rita O'Brien | Hillcrest Drive | Lucan | Co. Dublin |
| 5 | John Coleman | 6 Finnsgrove | Finnstown Cloisters | Lucan |
| 6 | Marie Tweedy | 14 Berryfield | Finnstown Priory | Lucan |
| 7 | Maria Grogan | None given | None given | None given |
| 8 | Deirdre Rooney | None given | None given | None given |
| 9 | Denise McNamara | None given | None given | None given |
| 10 | Christine Lynn | Finnscourt | Finnscourt Cloisters | Lucan |
| 11 | Patrick McManus | 17 Finnsview | Finnstown Cloisters | Lucan |
| 12 | Sandra Coleman | None given | None given | None given |
| 13 | Emily McMahon | 1 Woodville Close | Woodville Downs | Lucan |
| 14 | Barry Jenkinson | 1 Finnsgrove | Finnstown | Lucan |
| 15 | Patrick Leonard | 48 Esker Lawns | Lucan | Co. Dublin |
| 16 | Tom Power | 3 Finnsgreen | Lucan | Co. Dublin |
| 17 | Michael Foley | 7 Finnswood | Finnstown Cloisters | Lucan |
| 18 | Lesley Lennon | None given | None given | None given |
| 19 | Edina Almasi | None given | None given | None given |
| 20 | Karen Masterson | 6 Finns View | Finnstown Priory | Lucan |
| 21 | Aine Kinsella | None given | None given | None given |
| 22 | Andrew Breen | 6 Westbury Close | Lucan | Co. Dublin |
| 23 | Ann Leahy | None given | None given | None given |
| 24 | Ann Twohig | 6 Westbury Close | Lucan | Co. Dublin |
| 25 | Ann Gannon | None given | None given | None given |
| 26 | Ann Malone | None given | None given | None given |
| 27 | Avril Feeney | 9 Westbury Drive | Lucan | Co. Dublin |
| 28 | Barbara Ryan | None given | None given | None given |
| 29 | Barry Ryan | 5 Westbury Park | Lucan | Co. Dublin |
| 30 | Brenda Ui Mhaolhmana | 40 Westbury Drive | Lucan | Co. Dublin |
| 31 | Brian Murphy | Edmundsbury Court | Lucan | Co. Dublin |
| 32 | Caroline Monks | Fonthill Road | Lucan | Co. Dublin |
| 34 | Conor O'Shea | Woodville Estate | Lucan | Co. Dublin |
| 35 | Declan Dowling | 42 Westbury Avenue | Lucan | Co. Dublin |
| 37 | Denis Twohig | 30 Westbury Avenue | Lucan | Co. Dublin |
| 39 | Dermot McGreal | None given | None given | None given |
| 40 | Edward & Anne Flanagan | 46 Westbury Avenue | Lucan | Co. Dublin |
| 41 | Elaine Roche | 1 Fforester Row | Ballydowd Manor | Lucan |
| 42 | Elaine Twomey | 8 Westbury Close | Lucan | Co. Dublin |
| 43 | Else O'Neill | Woodville Downs | Lucan | Co. Dublin |
| 45 | Emma Mulligan | 11 Finnsview | Lucan | Co. Dublin |
| 46 | Fearghal Bracken | 97 Beechpark | Lucan | Co. Dublin |
| 47 | Frank Doyle | 11 Westbury Drive | Lucan | Co. Dublin |
| 48 | Gillian Farrell | None given | None given | None given |
| 49 | Grainne Hannon | None given | None given | None given |
| 50 | Greg Lonergan | 18 Westbury Avenue | Lucan | Co. Dublin |
| 51 | Hazel Smith | 1 Finnsgrove | Lucan | Co. Dublin |
| 52 | Ian McLaughlin | 13 Westbury Close | Lucan | Co. Dublin |
| 53 | Irene O'Shea | None given | None given | None given |
| 54 | Jacqui Molloy | None given | None given | None given |
| 55 | James O'Loughlin | 9 Finnsview | Finnstown | Lucan |
| 56 | Janet Dalton | Rossberry | Lucan | Co. Dublin |
| 57 | Jason Keogh | None given | None given | None given |
| 58 | Joe Whyte | None given | None given | None given |
| 62 | Lisa Mahon | None given | None given | None given |
| 63 | Lisa Scahill | 15 Woodville Walk | Lucan | Co. Dublin |
| 64 | Liz Cronin | Lucan Village | Lucan | Co. Dublin |
| 65 | Lori Clarke | None given | None given | None given |
| 66 | Louise & Declan McCabe | None given | None given | None given |
| 68 | Mark O'Connor | None given | None given | None given |
| 69 | Martin Finn | 12 Westbury Ave | Lucan | Co. Dublin |
| 70 | MaryKate Groark | None given | None given | Co. Dublin |
| 71 | Mary McCormack | 4 Willsbrook Drive | Lucan | Co. Dublin |
| 73 | Michael Malone | None given | None given | None given |
| 74 | Miriam Collins | Liffey Valley Park | Lucan | Co. Dublin |
| 75 | Nessa O'Rourke | 3 Beechpark | Lucan | Co. Dublin |
| 76 | Niall & Caroline Carabine | 42 Westbury Drive | Lucan | Co. Dublin |
| 77 | Nicola O'Dwyer | Elwood | Lucan | Co. Dublin |
| 78 | Pat O'Keeffe & Aine Kinsella | 3 Westbury Avenue | Lucan | Co. Dublin |
| 79 | Patricia & Stephen Byrne | 8 Westbury Grove | Lucan | Co. Dublin |
| 82 | Rafal Marzee | 3 Finnsview | Finnstown Cloisters | Co. Dublin |
| 83 | Ray Fox | None given | None given | None given |
| 84 | Remco Mayer | None given | None given | None given |
| 86 | Rosaleen Murphy | 24 Ballydowd Grove | Lucan | Co. Dublin |
| 88 | Sarah Comerford | 8 Ash Park Court | Lucan | Co. Dublin |
| 89 | Seamus Magann | 1 Fforester Row | Lucan | Co. Dublin |
| 90 | Seamus Small | Edmundsbury Court | Lucan | Co. Dublin |
| 92 | Sharon Gaynor | 7 Westbury Avenue | Lucan | Co. Dublin |
| 93 | Sinead Harrington | None given | None given | None given |
| 94 | Stephen Dempsey | 23 Westbury Drive | Lucan | Co. Dublin |
| 95 | Stephen McNally | 20 Westbury Ave | Lucan | Co. Dublin |
| 96 | Teresa & Pat Quinn | 5 Westbury Drive | Lucan | Co. Dublin |
| 97 | Teresa Gibbons | 4 Westbury Drive | Lucan | Co. Dublin |
| 98 | Terry Dent | 29 Westbury Drive | Lucan | Co. Dublin |
| 100 | Valerie & Patk McGreaney | 4 Finnsgrove | Finnstown Cloisters | Co. Dublin |
| 101 | William McSweeney | 37 Ardeeven Avenue | Lucan | Co. Dublin |
| 102 | Geraldine O'Toole | 27 Woodville Avenue | Lucan | Co. Dublin |
| 103 | Peter O'Toole | 27 Woodville Avenue | Lucan | Co. Dublin |
| 104 | Orla O'Toole | 27 Woodville Avenue | Lucan | Co. Dublin |
| 105 | Ciara O'toole | 27 Woodville Avenue | Lucan | Co. Dublin |
| 106 | Mary Garvey | 39 Ballydowd Grove | Lucan | Co. Dublin |
| 107 | Aideen Farrelly | 33 Lucan Heights | Lucan | Co. Dublin |
| 108 | Matt Farrelly | 33 Lucan Heights | Lucan | Co. Dublin |
| 109 | Josephine Farrelly | 33 Lucan Heights | Lucan | Co. Dublin |
| 110 | Sean Walsh | 30 Roselawn | Ballydowd Manor | Co. Dublin |
| 111 | Adrienne Molloy | 44 Lucan Heights | Lucan | Co. Dublin |
| 112 | Noel Molloy | 44 Lucan Heights | Lucan | Co. Dublin |
| 113 | Brendan Ryan | 7 Roselawn | Lucan | Co. Dublin |
| 115 | Ann Murphy | 43 Roselawn | Lucan | Co. Dublin |
| 116 | Tom Murphy | 43 Roselawn | Lucan | Co. Dublin |
| 117 | Inez Cooper | 6 Chalet Gardens | Lucan | Co. Dublin |
| 118 | Mary Murphy | 27 Roselawn | Lucan | Co. Dublin |
| 119 | John Dowling | Brook Cottage | Esker Lane | Lucan |
| 120 | Ann Malone | Lestram House | Main Road | Ballydowd |
| 121 | Roy Cooper | 6 Chalet Gardens | Lucan | Co. Dublin |
| 122 | Michael O'Halloran | 28 Roselawn | Lucan | Co. Dublin |
| 123 | Marie Flanagan | 25 Roselawn | Lucan | Co. Dublin |
| 124 | Jim & Chris Byrne | 252 Beech Park | Lucan | Co. Dublin |
| 125 | Teresa Duggan | 14 Hermitage Crescent | Hermitage Park | Lucan |
| 126 | Anne Connaughton | 48 Roselawn | Lucan | Co. Dublin |
| 127 | Elizabeth Allen | 108 Sarsfield Park | Lucan | Co. Dublin |
| 128 | Cait Cunningham | The Mileston | Old Lucan Road | Co. Dublin |
| 129 | Jenny Ni Dhoibhilin | 63 Sarsfield Park | Lucan | Co. Dublin |
| 130 | Maria Stephens | 12 Woodville Grove | Lucan | Co. Dublin |
| 131 | Margaret Maher | 14 Hermitage Manor | Lucan | Co. Dublin |
| 132 | Paul thompson | 1 Johnsbridge Grove | Lucan | Co. Dublin |
| 133 | Dolores Barrett | 17 Foxborough Park | Lucan | Co. Dublin |
| 134 | Sharon Thompson | 1 Johnsbridge Grove | Lucan | Co. Dublin |
| 135 | Michele McGowan | 8 Woodville Avenue | Lucan | Co. Dublin |
| 136 | Margaret Byrne | 256 Beech Park | Lucan | Co. Dublin |
| 137 | Orla Cunningham | 3 Esker Woods | Lucan | Co. Dublin |
| 138 | M Maher | 14 Hermitage Manor | Lucan | Co. Dublin |
| 139 | Eddie Connaughton | 48 Roselawn | Lucan | Co. Dublin |
| 140 | Sarah Cooper | 6 Chalet Gardens | Lucan | Co. Dublin |
| 141 | Donal Walsh | 9 Bewley Lawn | Lucan | Co. Dublin |
| 142 | Jim & Maria Copeland | 4 Brookfield | Lucan | Co. Dublin |
| 143 | David Solan | 4 Chalet Gardens | Lucan | Co. Dublin |
| 144 | Isobel Solan | 5 Chalet Gardens | Lucan | Co. Dublin |
| 145 | Ian Tighe | None given | None given | None given |
| 146 | Eamonn O'Brien | 11 Woodville Grove, | Lucan | Co Dublin |
| 147 | Helena O'Brien | 11 Woodville Grove, | Lucan | Co Dublin |
| 148 | Denis Kelly | None given | None given | None given |
| 149 | Michael Thurlow | Old Mill | Celbridge | Co Kildare |
| 150 | Rachel from Lucan | None given | None given | None given |
| 151 | Noel Corrigan | 8 Westbury Drive, | Lucan | Co Dublin |
| 152 | Ken Dunne | Sureweld International Ltd, | Lucan | Co Dublin |
| 153 | Woodville Downs Residents | None given | None given | None given |
| 154 | John Kelly | 3 Woodville Lawns | Lucan | Co Dublin |
| 155 | John Dominic O'Sullivan | None given | None given | None given |
| 156 | Anne Connaughton | None given | None given | None given |
| 157 | Paula Olmar | Woodville Close | Lucan | Co. Dublin |
| 158 | Bernie Grant | Woodville Avenue | Lucan | Co. Dublin |
| 159 | Keith Leonard | Beech Grove | Lucan | Co. Dublin |
| 160 | Margaret Leonard | Beech Grove | Lucan | Co. Dublin |
| 161 | Alice Neligan | Woodville Estate | Lucan | Co. Dublin |
| 162 | Teresa Brennan | 3 Rockfield | Finnstown Abbey | Co. Dublin |
| 163 | Laura Cunningham | Esker Pines | Lucan | Co. Dublin |
| 164 | Leanne Malone | None given | None given | None given |
| 165 | Paul Clarke | None given | None given | None given |
| 166 | Simon Thornton | None given | None given | None given |
| 167 | Eileen McCrory | 9 Ballydowd Grove | Lucan | Co. Dublin |
| 168 | Ann Hynes | 6 Woodville Avenue | Lucan | Co. Dublin |
| 169 | Ann Malone | None given | None given | None given |
| 170 | Frances Wright | Esker Lane | Lucan | Co. Dublin |
| 171 | Sylvia Dorigo | Esker Lane | Lucan | Co. Dublin |
| 172 | Sean Jones | 237 Beechpark | Lucan | Co Dublin |
| 173 | Raimonda & Sean Haughey | Woodville Walk | Lucan | Co. Dublin |
| 174 | Niall Faherty | Woodville Avenue | Lucan | Co. Dublin |
| 175 | John O'Connor | None given | None given | None given |
| 176 | Rosemary Skehan | None given | None given | None given |
| 177 | Binu K Philip | None given | None given | None given |
| 178 | Colette Carraher | None given | None given | None given |
| 179 | Anna O'Doherty | None given | None given | None given |
| 180 | Anne Hunston | None given | None given | None given |
| 181 | Mary Corrigan | None given | None given | None given |
| 182 | Ciara Dardis | None given | None given | None given |
| 183 | Michael Heffernan | None given | None given | None given |
| 184 | Irene O'Shea | None given | None given | None given |
| 185 | Adrienne Sullivan | 38 Westbury Avenue | Lucan | Co Dublin |
| 186 | Anne Keating | 66 Beech Park | Lucan | Co Dublin |
| 187 | Dr Gavin Kelly | None given | None given | None given |
| 188 | Breda & Ultan Tuite | 2 Woodville Grove | Lucan | Co Dublin |
| 189 | Teresa Halloran | Woodville Lawn | Lucan | Co Dublin |
| 190 | Colm Kelly | Woodville Downs | Lucan | Co Dublin |
| 191 | Ken Mulligan | None given | None given | None given |
| 192 | Brigid Mulqueen | None given | None given | None given |
| 193 | Paddy & Hillary Keating | 27 Ballydowd Grove | Lucan | Co Dublin |
| 194 | Shane Sharkey | Woodville Downs | Lucan | Co Dublin |
| 195 | Mark English | None given | None given | None given |