Top of Form

COMHAIRLE CONTAE ÁTHA CLIATH THEAS
SOUTH DUBLIN COUNTY COUNCIL

MEETING OF LUCAN AREA COMMITTEE

Tuesday, October 24, 2017

HEADED ITEM NO. 12

HEADED ITEM: K. Reilly

Traffic Modelling Report - Newcastle Road Interchange

REPLY:

Traffic Signal modelling of the Newcastle Interchange and Newcastle Rd/ Hillcrest Estate junctions was carried out by the Councils Traffic Section.

The traffic count data was obtained in March 2017 and the data for the AM peak (08:00 to 09:00) was used.

The signals were modelled using Network LinSig software developed by JCT Consultancy. This is an industry standard software.

The options modelled and the results are summarised below.

Option 1) Existing arrangement.

The existing signal arrangement was modelled as a reference base. The results show that the westbound off ramp had a degree of saturation of 84.3%, the right-turn from the bridge to the N4 city bound had a deg sat of 96.6% and the Hillcrest exit had a deg sat of 91.6%. It should be noted that a deg sat of over 90% is regarded as congested.

Option 2) 2- right-turning lanes.

An option allowing both northbound lanes on the interchange to turn right was modelled. As allow 2 lanes to turn right introduces a conflict with traffic exiting from Lucan Village an additional stage to accommodate this movement was required.

The model indicated that the deg sat of the right-turn increased to 121.9% and Hillcrest to 318.6 %. These are very highly congested levels.

Option 3) one ahead and 2 right turning lanes.

In this option the right-turn lane from Lucan village is partially removed and a short –right turn lane city bound introduced. This results in there being one ahead lane and two right turn lanes being available.

Modelling results indicate a deg sat on the right-turn of 116.2% and a deg sat at hillcrest of 213.8%. While these results are an improvement on Option 2 they are still considerably more congested that the existing arrangement.

Modelling indicates that the current signal arrangement is the best option currently available.

Bottom of Form