COMHAIRLE CHONTAE ÁTHA CLIATH THEAS
SOUTH DUBLIN COUNTY COUNCIL

MEETING OF TERRENURE/RATHFARNHAM AREA COMMITTEE (1)

TUESDAY 6TH JUNE 2006

HEADED ITEM NO. 3

Schools Traffic Management Issues  TERENURE\RATHFARNHAM ACM AREA

Background

The 2004 Road Traffic Act enables the Local authority to specify speed limits for various roads in the County. To-date the Council has carried out an overall review of speed limits and by adopting the appropriate Bye-Law introduced the current speed limits. The above Act also enables the Local Authority to introduce a 30 kph limit in certain locations, namely schools. In association with this, at the March 2006 Area Committee Meeting it was agreed to prioritise 2 schools in each electoral area for use as initial schemes under the Schools Survey heading. The schools for the Terenure/Rathfarnham area are as follows:

(a) Ballyboden

- Scoil an Spioraid Naomh

(b)Templeogue

- Bishop Galvin National School

- Bishop Shanahan National School

Each of the above has now been surveyed and assessed separately.

General

In recent years the Council has provided a range of traffic regulatory measures at various school locations. These include yellow lines, zig-zag lines, school ahead and children crossing signs. In general there is a school warden service at the majority of national and some secondary schools. Any further measures identified will complement the existing position.

Ballyboden

(1) Location:

The linked document identifies the location of the schools.

(2)Survey Details:

(a) The results of the initial school survey are shown on traffic management data sheets (see link) --- photos of problems associated with traffic congestion at school drop off/pick up times are also enclosed.

Ballyroan Scoil an Sporaid Naomh

(3)Problems Identified

- Very heavy traffic congestion on Ballyroan Road – Right turning traffic into Willowbank Drive adding to congestion

- Parents parking on path (school side) and the grass verge opposite the school obstructing school wardens

- Cars crossing pedestrian access to school door in order to access the car park inside the school grounds

- On sunny days motorists experience difficulty seeing the secondary signal on Ballyboden road and tend to break the lights

- Warning signage in poor condition, also may require relocation.

- A need for “School Keep Clear” markings

- Traffic Speed on Ballyroan Road not an issue

Preliminary Consultation.

A meeting took place with the school principal who identified the following issues:

- A need for parent parking opposite the school ie a dropping off zone

- Poor warning signage and insufficient road markings

- A need for bollards to be installed to prevent parking on the grass verge opposite the school

- Problems regarding pedestrian/vehicular access to the school

(4) Draft Recommendations

- To investigate the feasibility of drop-off parking outside school

- Installation of bollards along grass verge opposite the school

- An upgrade of school warning signage and road markings

- Consideration of a ban on the right turn into Willowbank Drive at school times

- The implementation of a traffic management plan within the school grounds

- The installation of directional hoods on the traffic lights.

- A reduction in the speed limit not recommended

Templeogue

(1) Location:

The linked document identifies the location of the schools.

Survey Details.

The results of the initial survey are shown on traffic management data sheets (see links) – photographs of problems associated with traffic congestion at school drop-off /pick-up times are included.

Bishop Galvin / Shanahans National School

Problems identified

- Heavy traffic congestion at school times – parents parking on both sides of both Rossmore Rd. and Orwell Park Rd. including on school keep clear markings and pedestrian zigzag (no parking) markings.

- Existing warning signage in poor condition.

- Lack of footpath dishing at crossing points.

- Ponding occurring at the pedestrian signals.

- Traffic speed is not a problem here.

Preliminary Consultation.

A meeting took place with the schools principal who identified the following issues

- Insufficient warning signage

- Too long a delay for pedestrians waiting to cross at lights

- No dishing on one side of the road at the school warden crossing point on Orwell Park Road.

Draft Recommendations.

- Upgrading of warning signage.

- Dishing of kerbs

- Improve drainage at the pedestrian crossing.

- Double yellow lining on the school side of both roads.

- The implementation of a “walk to school” initiative

Implementation

(a) Constraints

- Schools close for summer holidays at the end of June 2006

- Further consultation required with School Principals, Boards of Management, etc.

(b) Required

- Further Consultation

- Garda approval